|  | INTERNATIONAL TELECOMMUNICATION UNION TELECOMMUNICATION STANDARDIZATION SECTOR STUDY PERIOD 2025-2028 |  | SG17-Cn |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  | STUDY GROUP 17 |  |  |
|  |  |  | Original: English |  |  |
| Question(s): |  | [Q1/17, Q2/17, Q6/17, Q7/17, etc.] |  | Geneva, 8-17 April 2025 |  |
| CONTRIBUTION |  |  |  |  |  |
| Source: |  |  |  |  |  |
| Title: |  | It is a draft and not submitted to ITU |  |  |  |
| Contact: |  |  |  |  | Tel: E-mail: |
| Contact: |  |  |  |  | Tel: E-mail: |
| Contact: |  |  |  |  | Tel: E-mail: |
| Contact: |  |  |  |  | Tel: E-mail: |
| Contact: |  |  |  |  | Tel: E-mail: |

| Abstract: | [Currently, the industry has only standardized TLS-based security protocol for Modbus TCP communication. However, for EIA/TIA-485 multi-point systems, whether in 2-wire or 4-wire configurations, cables with a maximum baud rate of 9600 bit/s and AWG26 (or thicker) specifications can reach lengths over 1000m. For RS485 Modbus, sufficient cable diameter supports lengths over 1000m, and Category 5 cables can reach up to 600m. Despite its widespread use, Modbus serial link communication lacks standardized security mechanisms. As an application-layer industrial communication protocol, Modbus requires targeted security standards for different deployment scenarios to enhance its security framework and ecosystem. For instance, in scenarios involving hardware side-channel attacks or long-distance relay and bridged networks, transmitting plaintext Modbus data over serial links risks data leakage or malicious modification by intermediaries, posing significant data and command security threats. To address this, it is critical to improve the communication security of Modbus in serial link mode by introducing encryption and authentication mechanisms. A proposed Modbus serial link security standard defines simple encryption and authentication methods to significantly enhance security while maintaining compatibility, while maintaining compatibility. This provides a straightforward upgrade path for existing Modbus-based devices, ensuring secure and practical applications in industrial control systems.] |
| --- | --- |

- Background and motivation

  - The Security Status and Vulnerabilities of Modbus Protocol

Industrial control systems widely use the Modbus protocol for communication between devices, especially in the fields of manufacturing, energy, and automation. However, traditional Modbus protocol data is unencrypted, vulnerable to tampering and eavesdropping attacks, lacks authentication mechanisms, and is susceptible to unauthorized access. The Modbus protocol was first used in 1979 as a serial protocol for communication between industrial controllers and computers or other upper level computers. In 1996, the Modbus protocol was extended to Ethernet using IANA registered port 502, supporting Modbus TCP based Ethernet protocols. Meanwhile, Modbus TCP maintains consistency and compatibility with the Modbus RTU serial protocol, making it very easy for Modbus serial devices to achieve bridge communication through Modbus TCP. In 2002, Modbus abstracted the Modbus application layer applicable to serial and Ethernet, began to use the concept of Modbus protocol data unit (PDU), and released the Modbus application layer specification. For application data units (ADUs) different from serial bus and TCP, specifications for serial bus and Ethernet were also released. Meanwhile, Modbus has also incorporated IEC61784 as one of the industry standards. Based on the Modbus protocol specification in IEC 61784, China has released the GB/T19582 Modbus series recommended national standard. Also, due to the release of national standards, it is possible to conduct interoperability testing in accordance with regulatory standards. Later, the GB/T25919 series standards were formulated and released, which greatly improved the consistency and interoperability of a large number of Modbus applications, and was beneficial for the development and integration of industrial automation systems.

Like all industrial communication protocols, Modbus was not originally designed with information security features. The current security standards still mainly rely on patch based reinforcement, and there is no unified standard for upgrading native protocols. With the increasing demand for data confidentiality and integrity, device identification, and other requirements in industrial communication and applications, Modbus Security Specification and GB/T 41868-2022 standard has expanded the Modbus protocol using universal TLS transport layer encryption technology for Ethernet communication, enabling encrypted Modbus communication to increase its resistance to common attacks such as replay and man in the middle. Modbus TCP security maintains an ADU consistent with Modbus TCP, which enables easy migration of Modbus TCP communication to Modbus Security. Modbus TCP security uses port 802 registered by IANA for secure communication. Modbus security only allows the use of TLS1.2 and above versions. Modbus TCP security also uses X.509v3 digital signature certificate, which uses mutual authentication during TLS negotiation handshake between the client and server. At the same time, OID extension is used in the certificate, which allows device manufacturers to specify client roles and permissions, enabling user identification and role-based control required for industrial information security. And also, Modbus, IEC, NIST and ENISA, have mainly carried out standardization work on Modbus TCP and its management. For example Modbus Security Specification, IEC 62443 Series, IEC 62351 Series, NIST SP 800-82 and ENISA ICS Security Guidelines etc. And the Chinese national standards for Modbus protocol that have been released so far include: GB/T 19582.1-2008 "Industrial Automation Network Specification Based on Modbus Protocol Part 1: Modbus Application Protocol"; GB/T 19582.2-2008 "Industrial Automation Network Specification Based on Modbus Protocol Part 2: Implementation Guideline for Modbus Protocol on Serial Link"; GB/T 19582.3-2008 "Industrial Automation Network Specification Based on Modbus Protocol Part 3: Implementation Guideline for Modbus Protocol on TCP/IP";GB/T 25919.1-2010 "Modbus Test Specification Part 1: Modbus Serial Link Consistency Test Specification"; GB/T 25919.2-2010 "Modbus Test Specification Part 2: Modbus Serial Link Interoperability Test Specification"; GB/T 41868-2022 "Modbus TCP Security Protocol Specification"; These standards provide a comprehensive framework to strengthen Modbus security in industrial environments. However, In terms of security, these standards currently mainly establish protocol level specifications for Modbus TCP communication based on Ethernet. These standards are insufficient in terms of scenario application security. The serial-based Modbus communication remains vulnerable due to the lack of formalized security standards. Future work must focus on enhancing serial link security through lightweight encryption and authentication mechanisms, ensuring compatibility with existing systems while addressing modern security challenges.

  - Motivations

The Modbus protocol, widely used in industrial communication, was not originally designed with security in mind. Its lack of encryption, authentication, and data integrity mechanisms makes it vulnerable to various cyberattacks, including replay, man-in-the-middle (MITM), and unauthorized access. Currently, for any EIA/TIA-485 multi-point system, whether it is a 2-wire or 4-wire configuration, the maximum length for cables with a maximum baud rate of 9600bit/s and AWG26 (or thicker) specifications can reach over 1000m. For RS485 Modbus, a sufficiently wide cable diameter should allow for a maximum length of over 1000m, and for RS485 Modbus using Category 5 cables, the maximum length can reach 600m. Currently, there is no relevant security communication standard for the existing Modbus serial link communication method. As an industrial communication protocol at the application level, with the fast evolution of precision attack technology, Modbus needs to provide targeted security technology standards for different deployment scenarios to continuously improve the Modbus industrial application security guarantee system and ecology. For example, in the presence of modern advanced hardware side channel attacks, transmitting Modbus industrial control protocol data in plaintext over a fully serial link medium can easily lead to data leakage or malicious modification by intermediaries, especially in the case of long-distance relay serial or bridged networks, the case will also pose more serious data privacy or instruction security risks.

For Modbus TCP, significant progress has been made to address these vulnerabilities. The Modbus Security Specification introduces Transport Layer Security (TLS) to encrypt and authenticate communications. TLS 1.2 or higher is mandated, ensuring protection against eavesdropping and MITM attacks. Additionally, X.509v3 certificates enable mutual authentication between clients and servers, while role-based access control (RBAC) allows granular security configurations. The Chinese standard GB/T 41868-2022 has further expanded Modbus TCP security by adopting TLS encryption while maintaining compatibility with the original Application Data Unit (ADU) format. Modbus TCP Security uses IANA-registered port 802 for secure communication, providing a clear upgrade path for existing systems.

However, the security of serial link communication (e.g., RS485-based Modbus) remains an unresolved challenge. Serial Modbus transmits data in plaintext, leaving it susceptible to interception, modification, and hardware-based side-channel attacks. There are currently no formal standards addressing security for Modbus over RS485, which is still widely used due to its simplicity and long-distance capabilities. This lack of encryption and authentication mechanisms poses risks, particularly in scenarios involving relays or bridged networks. While Modbus TCP has seen notable advancements in security through TLS-based solutions, serial Modbus communication requires more attention. Introducing lightweight encryption and authentication mechanisms for serial links could provide a practical way to enhance security and protect legacy systems without significant infrastructure changes. Therefore, it is very necessary and valuable to improve and enhance the communication security of Modbus protocol under serial link mode, for reference by relevant institutions and organizations in various industries, in order to ensure the practical application security of various industrial control systems. With the introduction of the Modbus serial link security standard, the introduction of encryption and authentication mechanisms in the serial link communication channel of the Modbus protocol significantly improves its security, providing a relatively simple and direct upgrade path for existing devices that use Modbus extensively.

- Gap analysis

In terms of standardization work, international standardization organizations such as Modbus, IEC, NIST and ENISA, have mainly carried out standardization work on Modbus TCP. For example Modbus Security Specification, IEC 62443 Series, IEC 62351 Series, NIST SP 800-82 and ENISA ICS Security Guidelines etc. These standards provide a comprehensive framework to strengthen Modbus security in industrial environments. Despite these advancements, Modbus security remains a challenge due to its widespread use in legacy systems. Organizations must adopt these standards and consider integrating Modbus with more secure protocols like OPC UA to enhance security and considering the legacy systems. Implementing encryption, access control, and network segmentation is essential to mitigate risks in modern industrial environments. Like all industrial communication protocols, Modbus was not originally designed with information security features. Current security standards primarily focus on patch-based reinforcement, while a unified specification for upgrading the native protocol has yet to be established. As industrial communication and applications increasingly demand data confidentiality, integrity, and device identity verification, the existing standards have extended the Modbus protocol for Ethernet-based communication by adopting the general TLS (Transport Layer Security) encryption technology. This enhancement enables encrypted Modbus communication to better resist common attacks such as replay and man-in-the-middle attacks. Modbus TCP Security maintains compatibility with the Application Data Unit (ADU) of Modbus TCP, facilitating the seamless migration of Modbus TCP communication to Modbus Security. Secure Modbus TCP uses port 802, registered by IANA, for secure communication. Furthermore, Modbus Security mandates the use of TLS 1.2 or higher. It also incorporates X.509v3 digital certificates, implementing mutual authentication between clients and servers during the TLS handshake. The certificates utilize OID extensions, allowing device manufacturers to specify client roles and permissions, thereby enabling user identification and role-based access control as required for industrial information security. However, current standards stipulate that any EIA/TIA-485 multipoint system—whether in a 2-wire or 4-wire configuration—must support a maximum cable length of 1000m for baud rates up to 9600 bit/s using cables of AWG26 or thicker, also specify that for RS485-Modbus, the maximum allowable cable length should reach 1000m with sufficiently wide cable diameters, while the maximum length is 600m when using Category 5 cables. There are currently no security standards for the full Modbus serial link communication. As an application-layer industrial communication protocol, Modbus requires security standards tailored to different deployment scenarios to enhance its industrial application security framework and ecosystem. For instance, in scenarios involving hardware-based side-channel attacks, especially when long-distance relays or bridged networks are present, the plaintext transmission of Modbus industrial control protocol data over purely serial link media can lead to data leakage or malicious modification by intermediaries, posing significant risks to data and command security. It is therefore imperative to enhance the communication security of Modbus under serial link conditions and provide relevant technical standards for reference by industries and organizations, ensuring the operational security of industrial control systems. Overall, the security of the Modbus protocol has been improved through standards like the Modbus Security Specification, IEC 62443, and GB/T 41868-2022, especially for Ethernet-based Modbus TCP. However, securing Modbus over serial links remains a challenge, with no formal standards yet addressing this area comprehensively. Future developments must focus on enhancing security for serial-based communication while maintaining compatibility with existing Modbus systems.

    - IEC 62443-3-3:2013: System Security Requirements and Security Levels

Summary:	This standard specifies system-level security requirements for Industrial Automation and Control Systems (IACS). It includes guidelines for access control, authentication, data integrity, and monitoring, which are applicable to securing Modbus-based systems.

Analysis:	These standards defines the comprehensive cybersecurity frameworks for industrial control systems (ICS), outlines system-level security requirements. However, this specification has no native security consideration for the pure Modbus serial link communication.

    - IEC 62443-4-2:2019: Technical Security Requirements for IACS Components

Summary:	This standard defines technical security requirements for industrial control devices, including secure communication protocols, encryption, and authentication mechanisms, relevant for improving the security of devices using Modbus.

Analysis:	This standard focuses on device-level protections, including secure communication protocols and defines technical security requirements for industrial control devices, including secure communication protocols, encryption, and authentication mechanisms, relevant for improving the security of devices using Modbus. However, this specification has no native security consideration for the pure Modbus serial link communication.

    - IEC 62351-3:2014: Power Systems Management and Associated Information Exchange – Data and Communications Security – Part 3: Communication Network and System Security for Profiles Including TCP/IP

Summary:	This standard provides securing communication networks that use TCP/IP, including Modbus TCP. It recommends encrypting communication and implementing authentication to mitigate cyber threats.

Analysis:	This standard focuses on the securing communication networks that use TCP/IP, including Modbus TCP. It provide encryption and authentication guidelines for protocols like Modbus TCP. However, this specification has no native security consideration for the pure Modbus serial link communication.

    - IEC 62351-4:2018: Power Systems Management and Associated Information Exchange – Data and Communications Security – Part 4: Profiles Including MMS and Derivatives

Summary:	This standard focuses on the security concerns for TCP/IP-based communication protocols, including Modbus TCP, and provides mechanisms for authentication and data integrity.

Analysis:	This standard addresses the security concerns for TCP/IP-based communication protocols, including Modbus TCP, and provides mechanisms for authentication and data integrity. However, this specification has no native security consideration for the pure Modbus serial link communication.

    - Modbus Security Specification: Modbus Application Protocol Specification v1.1b3 (with Security Extensions)

Summary:	This standard is developed by the Modbus Organization, this specification extends the Modbus protocol by adding Transport Layer Security (TLS) encryption. It includes: TLS 1.2 or higher: Ensures encrypted and authenticated communication. X.509v3 Certificates: Enables mutual authentication between devices. Role-Based Access Control: Implements role and permission differentiation using OID extensions in certificates. Modbus Security maintains compatibility with Modbus TCP and uses IANA-registered port 802 for secure communication.

Analysis:	This standard enhances Modbus TCP with security features like TLS encryption, message authentication, and endpoint authentication with X.509v3 Certificates: Enables mutual authentication between devices. Role-Based Access Control: Implements role and permission differentiation using OID extensions in certificates. Modbus Security maintains compatibility with Modbus TCP and uses IANA-registered port 802 for secure communication. However, this specification has no native security consideration for the pure Modbus serial link communication.

    - NIST SP 800-82: Guide to Industrial Control Systems (ICS) Security

Summary:	This standard is published by the National Institute of Standards and Technology (NIST), this guideline provides recommendations for securing industrial control systems. For Modbus, it highlights: Using VPNs and encryption technologies such as TLS. Implementing network segmentation and access control policies. Monitoring and detecting unauthorized Modbus traffic.

Analysis:	This U.S. guideline highlights best practices for ICS security, including encrypting Modbus traffic, network segmentation, and using Virtual Private Networks (VPNs). However, this specification has no native security consideration for the pure Modbus serial link communication.

    - ENISA ICS Security Guidelines: Good Practices for Security of Industrial Control Systems

Summary:	These European guidelines provide practical recommendations for securing industrial protocols, including Modbus. Key suggestions include: Monitoring and analysing Modbus network traffic. Implementing intrusion detection systems (IDS). Mitigating risks associated with plaintext Modbus communication.

Analysis:	These guidelines focuses on the practical recommendations for securing industrial protocols, including Modbus. Key suggestions include: Monitoring and analysing Modbus network traffic. Implementing intrusion detection systems (IDS). Mitigating risks associated with plaintext Modbus communication. However, this specification has no native security consideration for the pure Modbus serial link communication.

    - ANSI/ISA-99 Series, ANSI/ISA-99.00.01-2007: Security for Industrial Automation and Control Systems

Summary:	This series provides a framework for securing industrial systems, including Modbus networks. It emphasizes network segmentation, secure access control, firewalls, and intrusion detection to protect Modbus-based systems.

Analysis:	This series focuses on a framework for securing industrial systems, including Modbus networks. It emphasizes network segmentation, secure access control, firewalls, and intrusion detection to protect Modbus-based systems. However, this specification has no native security consideration for the pure Modbus serial link communication.

    - GB/T 41868—2022 Modbus TCP security protocol specification

Summary:	This standard provides the Modbus security protocol over Ethernet TCP. It highlights communication requirements, such as requiring PKI certificate format, TLS2.0 protocol, Crypto Suite selection, and more.

Analysis:	This standard provides the Modbus security protocol over Ethernet TCP. It highlights communication requirements, such as requiring PKI certificate format, TLS2.0 protocol, Crypto Suite selection and more, but currently lacks security mechanisms for serial-based communication.

- Proposal

This contribution proposes to initiate a new work item at SG17 on “X.MBSL-sec: Secure Extension of Modbus Serial Link Communication Security Protocol Reference Specification and Implementation Guide”.

In this work item, it is mainly intended to provide the extended security service definition of Modbus serial bus, protocol specifications, system dependencies, and corresponding requirements for the Modbus serial link communication security protocol. This draft recommendation is applicable to institutions and organizations involved in the development or testing of Modbus products.

- Annex

- Annex I: A.1 justification for proposed new work item: “X.MBSL-sec: Modbus Serial Link Communication Security Protocol Reference Specification and Implementation Guide”

- Annex II: Draft Recommendation

- Annex III: Parameter number table exchanged in APDU

- Annex IV: Pseudo-random number generator

- Annex V: Trust Model and Certificate Format

Annex 1
A.1 justification for Proposal for initiating a new work item ITU-T X.MBSL-sec: “Modbus Serial Link Communication Security Protocol Reference Specification and Implementation Guide”

| Question: | Q6/17 | Proposed new ITU-T Recommendation | Geneva, 8 April – 17 April 2025 |  |
| --- | --- | --- | --- | --- |
| Reference and title: | ITU-T X.MBSL-sec: Secure Extension of Modbus Serial Link Communication Protocol Reference Specification and Implementation Guide |  |  |  |
| Base text: | Annex II |  | Timing: | April, 2027 |
| Editor(s): |  |  | Approval process: | AAP |
| Scope (defines the intent or object of the Recommendation and the aspects covered, thereby indicating the limits of its applicability): |  |  |  |  |
| This draft recommendation aims to provide the extended security service definition of Modbus serial bus, protocol reference specifications, system dependencies, and corresponding requirements guideline for the Modbus serial link communication security protocol. This draft recommendation is applicable to institutions and organizations involved in the development or testing of Modbus products. |  |  |  |  |
| Summary (provides a brief overview of the purpose and contents of the Recommendation, thus permitting readers to judge its usefulness for their work): |  |  |  |  |
| The Modbus protocol, widely used in industrial communication, was not originally designed with security in mind. Its lack of encryption, authentication, and data integrity mechanisms makes it vulnerable to various cyberattacks, including replay, man-in-the-middle (MITM), and unauthorized access. Currently, for any EIA/TIA-485 multi-point system, whether it is a 2-wire or 4-wire configuration, the maximum length for cables with a maximum baud rate of 9600bit/s and AWG26 (or thicker) specifications can reach over 1000m. For RS485 Modbus, a sufficiently wide cable diameter should allow for a maximum length of over 1000m, and for RS485 Modbus using Category 5 cables, the maximum length can reach 600m. Currently, there is no relevant security communication standard for the existing Modbus serial link communication method. As an industrial communication protocol at the application level, Modbus needs to provide targeted security technology standards for different deployment scenarios to continuously improve the Modbus industrial application security guarantee system and ecology. For example, in the presence of hardware side channel attacks, especially in the case of long-distance relay serial or bridged networks, transmitting Modbus industrial control protocol data in plaintext over a fully serial link medium can easily lead to data leakage or malicious modification by intermediaries, posing serious data or instruction security risks. While Modbus TCP has seen notable advancements in security through TLS-based solutions, serial Modbus communication requires more attention. Introducing lightweight encryption and authentication mechanisms for serial links can provide a practical way to enhance security and protect legacy systems without requiring significant infrastructure changes, while maintaining compatibility. Therefore, it is meaningful to improve and enhance the communication security of Modbus protocol under serial link mode, for reference by relevant institutions and organizations in various industries, in order to ensure the practical application security of various industrial control systems. With the introduction of the Modbus serial link security standard, the introduction of encryption and authentication mechanisms in the serial link communication channel of the Modbus protocol significantly improves its security, providing a relatively simple and direct upgrade path for existing devices that use Modbus extensively. |  |  |  |  |
| Relations to ITU-T Recommendations or to other standards (approved or under development): |  |  |  |  |
| In terms of standardization work, international standardization organizations such as Modbus, IEC, NIST and ENISA, have mainly carried out several standardization works on Modbus TCP. For example Modbus Security Specification, IEC 62443 Series, IEC 62351 Series, NIST SP 800-82 and ENISA ICS Security Guidelines etc. These standards provide a comprehensive framework to strengthen Modbus security in industrial environments via TCP communications and its management. For serial bus communications, Modbus security remains a challenge due to its widespread use in legacy systems via serial bus protocol. The current standards do not address the risk problems in modern industrial environments. |  |  |  |  |
| Liaisons with other study groups or with other standards bodies: |  |  |  |  |
| IEC, Modbus Organization |  |  |  |  |
| Supporting members that are committing to contributing actively to the work item: |  |  |  |  |
| China Mobile, Harbin Institute of Technology, Huawei, ZTE, Schneider Electric (China) Co., Ltd, CAICT，H3C |  |  |  |  |

- Annex II

Draft Recommendation ITU-T X.MBSL-sec

Secure Extension of Modbus Serial Link Communication Security Protocol Reference Specification and Implementation Guide

Summary

This draft recommendation provides the extended security service definition of Modbus serial bus, protocol specifications, system dependencies, and corresponding requirements for the Modbus serial link communication security protocol. This draft recommendation is applicable to institutions and organizations involved in the development or testing of Modbus products.

Keywords

Modbus protocol, Serial bus communication, Security.

- Scope

This draft Recommendation aims to study the security protocol specification for Modbus serial bus communication.

The scope of this draft recommendation includes:

- Modbus serial link communication security service definition,

- Protocol reference specifications,

- System dependencies, guideline, including trust model and certificate format etc.,

- Security considerations.

- References

The following ITU-T Recommendations and other references contain provisions which, through reference in this text, constitute provisions of this Recommendation. At the time of publication, the editions indicated were valid. All Recommendations and other references are subject to revision; users of this Recommendation are therefore encouraged to investigate the possibility of applying the most recent edition of the Recommendations and other references listed below. A list of the currently valid ITU-T Recommendations is regularly published.

The reference to a draft recommendation within this Recommendation does not give it, as a stand-alone draft recommendation, the status of a Recommendation.

[GB/T 16262.1-2006] Recommendation ITU-T X.500 | ISO/IEC 9594-1, Information Technology-Open System Interconnection-The Directory: Overview of concepts, models and services.

[ITU-T X.501] Recommendation ITU-T X.501 | ISO/IEC 9594-2, Information Technology-Open System Interconnection-The Directory: Models.

[ITU-T X.509] Recommendation ITU-TX.509 | ISO/IEC 9594-8, Information Technology-Open System Interconnection-The Directory: Public-key and attribute certificate framework.

[GB/T 16262.1-2006] Information Technology Abstract Syntax Notation 1 (ASN. 1) Part 1: Basic Notation Specification

[GB/T 16264.8-2005] Information Technology - Open Systems Interconnection Catalog - Part 8: Public Key and Attribute Certificate Framework

[GB/T 20518] Information Security Technology Public Key Infrastructure Digital Certificate Format

[IETF RFC 5280] Internet X.509 Public Key Infrastructure Certificate and Certificate Revocation List (CRL) Profile

[FIPS 197-2001] Advanced Encryption Standard

[RFC3566-2003] AES-XCBC-MAC-96 Algorithm and Its Use with IPsec

[BS ISO/IEC 15946-1-2003] Information technology - Security techniques - General principles for elliptic curve cryptography

[BS ISO/IEC 15946-2:2003] Information technology security techniques - Cryptography techniques based on elliptic curves - Digital signatures

[BS ISO/IEC 15946-3-2003] Information technology security techniques - Cryptography based on elliptic curves - Determination of keys

[ISO/IEC 14888-3:2018] IT Security techniques — Digital signatures with appendix — Part 3: Discrete logarithm based mechanisms

[FIPS 140-2-2002] Security Requirements for Cryptographic Modules

[FIPS 180-2-2002] Secure Hash Standard

[IEEE 1363-2000] IEEE standard specifications for public key cryptography

[GM/T 0006] Password Application Identification Specification

[GM/T BBBBB] SM2 Password Algorithm Usage Specification

[GM/T CCCCC] SM2 Cryptography Algorithm Encryption Signature Message Syntax Specification

[GB/T 25056] Information Security Technology Certificate Certification System Code and Related Security Technology Specifications

[GB/T 32905] Information Security Technology SM3 Password Hash Algorithm

[GB/T 32907] Information Security Technology SM4 Block Key Algorithm

[GB/T 32918] Information Security Technology SM2 Circular Curve Public Key Algorithm

[GB/T 36624] Information Technology - Security Techniques - Verifiable Encryption Mechanisms

[IEC 62443-3-3:2013] System Security Requirements and Security Levels

[IEC 62443-4-2:2019] Technical Security Requirements for IACS Components

[SCTE 41:2004] POD copy protection system. Society of Cable Telecommunications Engineers.

http://www.scte.org/documents/pdf/ANSISCTE412004.pdf

[ANSI X 9.31:1998] American National Standards Institute, Digital Signatures using reversible public key cryptography for financial services industry (rDSA).

[ETSI/SAGE TS35.221:2020] Specification of the 3GPP Confidentiality and Integrity Algorithms 128-EEA3 &.128-EIA3.Document 1:128-EEA3 and 128-EIA3 Specification

[ETSI/SAGE TS35.222:2024] Specification of the 3GPP Confidentiality and Integrity Algorithms 128-FEA3 & 128-EIA3.Document 2: ZUC Specification

[GB/T 33133.1-2016] Information security technology ZUC stream cipher algorithm Part 1: Algorithm description

[GB/T 33133.2-2021] Information security technology ZUC stream cipher algorithm Part 2: Confidentiality algorithm

[GB/T 33133.2-2021] Information security technology ZUC stream cipher algorithm Part 3: Integrity algorithm

[IETF RFC 8439] ChaCha20 and Poly1305 for IETF Protocols

[NIST.FIPS.203] Module-Lattice-Based Key-Encapsulation Mechanism Standard

[NIST.FIPS.204] Module-Lattice-Based Digital Signature Standard

- Definitions

  - Terms defined elsewhere

This Recommendation uses the following terms defined elsewhere:

3.1.1 Root certificate [b-ITU-T X.509]: A root certificate is a certificate issued by a Root Certificate Authority (CA) to itself, serving as the starting point of the trust chain and representing the top of the certificate trust chain. The root certificate contains the public key and certificate signature of the root CA, used to sign and verify all certificates issued by that CA or its subordinate intermediate CAs.

3.1.2 Intermediate certificate [b-ITU-T X.509]: The intermediate certificate is a certificate obtained by the root certificate authority CA digitally signing the public key of the intermediate certificate authority (ICAA), and is located in the intermediate link of the certificate chain between the root certificate and the server certificate. Intermediate certificates are used to establish a trust chain from the root certificate to the server certificate.

3.1.3 PKI [b-ITU-T X.509]: In public key infrastructure, an information system based on public key cryptography that enables the issuance, revocation, and management of digital certificates, and provides corresponding services to subscribers..

3.1.4 Public key certificate [b-ITU-T X.509]: The user's public key, along with other information, is signed by the private key of the certificate authority that issued the certificate to make it unforgeable.

3.1.5 Trust anchor [b-ITU-T X.509]: An entity that is trusted by a relying party and used for validating public-key certificates.

3.1.6 Certification authority [b-ITU-T X.509]: An authoritative organization trusted by users and responsible for creating and distributing certificates. Certificate authorities can also create keys for users.

3.1.7 RS232 bus [b-IEC 62443]: A data communication bus implemented based on the EIA/TIA-232 standard. The RS-232 standard interface (also known as EIA RS-232) is one of the commonly used serial communication interface standards. It was jointly developed by the Electronic Industry Association (EIA), Bell Systems, modem manufacturers, and computer terminal manufacturers in 1970. Its full name is "Technical Standard for Serial Binary Data Exchange Interface between Data Terminal Equipment (DTE) and Data Communication Equipment (DCE)".

3.1.8 RS485 bus [b-IEC 62443]: A data communication bus implemented based on the EIA/TIA-485 standard. EIA-485 (formerly known as RS-485 or RS485) is a standard for electrical characteristics specified as 2-wire, half duplex, balanced transmission line multipoint communication belonging to the OSI model physical layer. It is a standard jointly released by the Telecommunications Industry Association (TIA) and the Electronic Industries Alliance (EIA). The digital communication network that implements this standard can efficiently communicate over long distances in environments with electronic noise. In a linear multipoint bus configuration, multiple receivers can be present on a network.

  - Terms defined in this Recommendation

This Recommendation defines the following terms:

3.2.1 Trust anchor: An entity that is trusted by relying parties to validate the ownership, delegation, and authenticity of network resources. It serves as a root of trust for establishing and verifying resource allocation chains across multiple infrastructure services through cryptographic mechanisms and distributed consensus.

3.2.2 Modbus device: A device that implements Modbus on a serial link and follows its technical specifications.

3.2.3 Secure authenticated channel: A logical channel established using password methods between the master and slave stations that supports authentication and encryption of data.

3.2.4 Nonce: A randomly selected one-time value used to add to a message or protocol to prevent replay attacks.

3.2.5 Encrypt: Process data using specific algorithms to prevent unauthorized access.

3.2.6 Master client: The master client is a device on the serial communication link that sends instruction request and there is usually only one such device on a serial communication link.

3.2.7 Slave server: The slave client is a device on the serial communication link that sends instruction response. Generally, there can be multiple such devices on a single serial communication link.

- Abbreviations and acronyms

This Recommendation uses the following abbreviations and acronyms:

AES		Advanced Encryption Standard

ADU		Application Data Unit

AKH		Authentication key Host

APDU		Application Protocol Data Unit

AuthZ		Authorization

ASN.1		Abstract Syntax Notation One

BCK		Broadcast Content Key

BER		Basic Encoding Rules

Bslbf		Bit serial leftmost bit first

CA		Certificate Authority

CBC		Cipher Block Chaining

CDP		CRL Distribution Point

CK		Content Key

CRL		Certificate Revocation List

DCE		Data Communication Equipment

DH		Diffie-Hellman key exchange

DHSK		Diffie-Hellman session key

DTE		Data Terminal Equipment

DN		Distinguished name

ECC		Elliptic Curves Cryptography

HMAC	Keyed-hash Message Authentication Code

HKDF		HAMC-based Key Derivation Function

IANA		Internet Assigned Numbers Authority

ICS		Industrial Control System

IV		Initialisation Vector

LPDU		Link Protocol Data Unit

LSB		Least significant bit

MAC		Message Authentication Code

MBAP		Modbus Application Protocol

MBAPS	Modbus Security Application Protocol

NVM		Non-volatile memory

OTP		One-Time Programmable

PDU		Protocol Data Unit

PKI		Public Key Infrastructure

PLC		Programmable Logic Controller

PRF		Pseudorandom Function Family

ROT		Root Of Trust (i.e. Trust Authority)

RS-232	Recommended Standard-232

RS-485	Recommended Standard-485

SAC		Secure Authenticated Channel

SAK		SAC Authentication Key

SM3		Secure Hash Algorithm 256-bit

SEK		SAC Encryption Key

SHA256	Secure Hash Algorithm 256-bit

SIV		SAC Initialisation Vector

SSL		Secure Socket Layer

TCP		Transport Control Protocol

TLS		Transport Layer Security

Uimsbf	Unsigned integer most significant bit first

ZUC                ZU Chongzhi

- Conventions

In this draft Recommendation:

The keywords "shall/necessary/is required to" indicate a requirement which must be strictly followed and from which no deviation is permitted, if conformance to this Recommendation is to be claimed.

The keywords "should/recommend" indicate a requirement which is recommended but which is not absolutely required. Thus, this requirement need not be present to claim conformance.

The keywords "can/optional" indicate an optional requirement which is permissible, without implying any sense of being recommended. This term is not intended to imply that the vendor's implementation must provide the option, and the feature can be optionally enabled by the network operator/service provider. Rather, it means the vendor may optionally provide the feature and still claim conformance with this Recommendation.

- Background and motivations

  - The Security Status and Vulnerabilities of Modbus Protocol

Industrial control systems widely use the Modbus protocol for communication between devices, especially in the fields of manufacturing, energy, and automation. However, traditional Modbus protocol data is unencrypted, vulnerable to tampering and eavesdropping attacks, lacks authentication mechanisms, and is susceptible to unauthorized access. The Modbus protocol was first used in 1979 as a serial protocol for communication between industrial controllers and computers or other upper level computers. In 1996, the Modbus protocol was extended to Ethernet using IANA registered port 502, supporting Modbus TCP based Ethernet protocols. Meanwhile, Modbus TCP maintains consistency and compatibility with the Modbus RTU serial protocol, making it very easy for Modbus serial devices to achieve bridge communication through Modbus TCP. In 2002, Modbus abstracted the Modbus application layer applicable to serial and Ethernet, began to use the concept of Modbus protocol data unit (PDU), and released the Modbus application layer specification. For application data units (ADUs) different from serial bus and TCP, specifications for serial bus and Ethernet were also released. Meanwhile, Modbus has also incorporated IEC61784 as one of the industry standards. Based on the Modbus protocol specification in IEC 61784, China has released the GB/T19582 Modbus series recommended national standard. Also, due to the release of national standards, it is possible to conduct interoperability testing in accordance with regulatory standards. Later, the GB/T25919 series standards were formulated and released, which greatly improved the consistency and interoperability of a large number of Modbus applications, and was beneficial for the development and integration of industrial automation systems.

Like all industrial communication protocols, Modbus was not originally designed with information security features. The current security standards still mainly rely on patch based reinforcement, and there is no unified standard for upgrading native protocols. With the increasing demand for data confidentiality and integrity, device identification, and other requirements in industrial communication and applications, Modbus Security Specification and GB/T 41868-2022 standard has expanded the Modbus protocol using universal TLS transport layer encryption technology for Ethernet communication, enabling encrypted Modbus communication to increase its resistance to common attacks such as replay and man in the middle. Modbus TCP security maintains an ADU consistent with Modbus TCP, which enables easy migration of Modbus TCP communication to Modbus Security. Modbus TCP security uses port 802 registered by IANA for secure communication. Modbus security only allows the use of TLS1.2 and above versions. Modbus TCP security also uses X.509v3 digital signature certificate, which uses mutual authentication during TLS negotiation handshake between the client and server. At the same time, OID extension is used in the certificate, which allows device manufacturers to specify client roles and permissions, enabling user identification and role-based control required for industrial information security. And also, Modbus, IEC, NIST and ENISA, have mainly carried out standardization work on Modbus TCP and its management. For example Modbus Security Specification, IEC 62443 Series, IEC 62351 Series, NIST SP 800-82 and ENISA ICS Security Guidelines etc. And the Chinese national standards for Modbus protocol that have been released so far include: GB/T 19582.1-2008 "Industrial Automation Network Specification Based on Modbus Protocol Part 1: Modbus Application Protocol"; GB/T 19582.2-2008 "Industrial Automation Network Specification Based on Modbus Protocol Part 2: Implementation Guideline for Modbus Protocol on Serial Link"; GB/T 19582.3-2008 "Industrial Automation Network Specification Based on Modbus Protocol Part 3: Implementation Guideline for Modbus Protocol on TCP/IP";GB/T 25919.1-2010 "Modbus Test Specification Part 1: Modbus Serial Link Consistency Test Specification"; GB/T 25919.2-2010 "Modbus Test Specification Part 2: Modbus Serial Link Interoperability Test Specification"; GB/T 41868-2022 "Modbus TCP Security Protocol Specification"; These standards provide a comprehensive framework to strengthen Modbus security in industrial environments. However, In terms of security, these standards currently mainly establish protocol level specifications for Modbus TCP communication based on Ethernet. These standards are insufficient in terms of scenario application security. The serial-based Modbus communication remains vulnerable due to the lack of formalized security standards. Future work must focus on enhancing serial link security through lightweight encryption and authentication mechanisms, ensuring compatibility with existing systems while addressing modern security challenges.

  - Motivations

The Modbus protocol, widely used in industrial communication, was not originally designed with security in mind. Its lack of encryption, authentication, and data integrity mechanisms makes it vulnerable to various cyberattacks, including replay, man-in-the-middle (MITM), and unauthorized access. Currently, for any EIA/TIA-485 multi-point system, whether it is a 2-wire or 4-wire configuration, the maximum length for cables with a maximum baud rate of 9600bit/s and AWG26 (or thicker) specifications can reach over 1000m. For RS485 Modbus, a sufficiently wide cable diameter should allow for a maximum length of over 1000m, and for RS485 Modbus using Category 5 cables, the maximum length can reach 600m. Currently, there is no relevant security communication standard for the existing Modbus serial link communication method. As an industrial communication protocol at the application level, with the fast evolution of precision attack technology, Modbus needs to provide targeted security technology standards for different deployment scenarios to continuously improve the Modbus industrial application security guarantee system and ecology. For example, in the presence of modern advanced hardware side channel attacks, transmitting Modbus industrial control protocol data in plaintext over a fully serial link medium can easily lead to data leakage or malicious modification by intermediaries, especially in the case of long-distance relay serial or bridged networks, the case will also pose more serious data privacy or instruction security risks.

For Modbus TCP, significant progress has been made to address these vulnerabilities. The Modbus Security Specification introduces Transport Layer Security (TLS) to encrypt and authenticate communications. TLS 1.2 or higher is mandated, ensuring protection against eavesdropping and MITM attacks. Additionally, X.509v3 certificates enable mutual authentication between clients and servers, while role-based access control (RBAC) allows granular security configurations. The Chinese standard GB/T 41868-2022 has further expanded Modbus TCP security by adopting TLS encryption while maintaining compatibility with the original Application Data Unit (ADU) format. Modbus TCP Security uses IANA-registered port 802 for secure communication, providing a clear upgrade path for existing systems.

However, the security of serial link communication (e.g., RS485-based Modbus) remains an unresolved challenge. Serial Modbus transmits data in plaintext, leaving it susceptible to interception, modification, and hardware-based side-channel attacks. There are currently no formal standards addressing security for Modbus over RS485, which is still widely used due to its simplicity and long-distance capabilities. This lack of encryption and authentication mechanisms poses risks, particularly in scenarios involving relays or bridged networks. While Modbus TCP has seen notable advancements in security through TLS-based solutions, serial Modbus communication requires more attention. Introducing lightweight encryption and authentication mechanisms for serial links could provide a practical way to enhance security and protect legacy systems without significant infrastructure changes. Therefore, it is very necessary and valuable to improve and enhance the communication security of Modbus protocol under serial link mode, for reference by relevant institutions and organizations in various industries, in order to ensure the practical application security of various industrial control systems. With the introduction of the Modbus serial link security standard, the introduction of encryption and authentication mechanisms in the serial link communication channel of the Modbus protocol significantly improves its security, providing a relatively simple and direct upgrade path for existing devices that use Modbus extensively.

- Overview of Modbus Serial Link Protocol Principles and Background

According to the definition in the standard GB/T 19582.2-2008 and IEC 61158 CPF15 (FDIS)-2006, the Modbus serial link communication protocol based on EIA/TIA-485-A and EIA/TIA-232-E is a widely used master-slave application layer protocol in industrial control systems. At present, the national standard requires any EIA/TIA-485 multi-point system, whether it is a 2-wire configuration or a 4-wire configuration, to have a maximum length of 1000m for cables with a maximum baud rate of 9600bit/s and AWG26 (or thicker) specifications. GB/T 19582.2-2008 also specifies that for RS485 Modbus, a sufficiently wide cable diameter should allow for a maximum length of 1000m, and for RS485 Modbus using Category 5 cables, the maximum length can reach 600m. At the same time, there can only be one master station and one or more (up to 247) slave stations on the serial link bus. Modbus communication is always initiated by the master station. When the slave station does not receive a request from the master station, it will not send any data and cannot communicate with each other. In addition, the master station can only initiate one Modbus transaction at a time, and can only start the next transaction after one transaction is completed.

Figure 1: Modbus Serial Link Communication Network

The master station generally sends Modbus requests to the slave stations through two working modes: unicast and broadcast. Among them, in unicast mode, the master station addresses a single slave station, and after the slave station receives and processes the request, it returns a message to the master station as a "response". A Modbus transaction consists of two messages: one is a request from the master station, and the other is a response from the slave station. Each slave station must have a unique address (1-247) in order to be addressed independently from other stations. In broadcast mode, when the broadcast address is 0, the master station can send requests to all slave stations with write commands, and the slave stations do not need to return a response.

The Modbus master station does not have a specific addressing address, and the Modbus addressing space is used to define the slave stations, consisting of 256 different addresses. Address 0 is the broadcast address, and all slave stations must recognize the broadcast address. The currently defined effective Modbus addressing space is 1-247, and the reserved addressing space is 248-255. Only the slave station has one address and the address must be unique on the Modbus serial bus.

The Modbus application protocol defines a simple protocol data unit PDU (Modbus PDU) that is independent of lower level communication. Modbus serial link communication defines Modbus serial link frames by adding address fields and CRC/LRC checksum fields based on this PDU. The definition of Modbus serial link frame format is shown in Figure 2 below:

Figure 2: Modbus Serial Link Frame

The master station addresses the slave station by placing its address in the Modbus serial link frame address field. When a slave station returns a response, it puts its address in the response address field so that the master station knows which slave station is responding. The function code indicates what operation the server should perform. After the function code is a data field containing request or response parameters. The CRC or LRC error check fields correspond to the serial RTU or ASCII transmission mode, respectively, and their values are calculated based on the redundancy check performed on the message content. On the Modbus serial link, the transmission mode (and serial port parameters) of all devices must be the same. Although ASCII mode is required in certain specific applications, interoperability between Modbus devices can only be achieved if each device has the same mode: all devices must implement RTU serial transmission mode, with ASCII serial transmission mode being an optional option. Users should set the device to the desired mode: RTU or ASCII mode, and the default setting must be RTU mode.

Each 8-bit byte in the Modbus RTU mode message frame contains two 4-bit hexadecimal characters. The main advantage of this mode is that it has a higher character density and higher data throughput than ASCII mode at the same baud rate. In RTU transmission mode, each byte occupies 11 bits when sending a frame, and its format is: 1 start bit; 8 data bits, first send the least significant bit; 1 parity bit; 1 stop position. By default, even parity is used, but it can also be left unchecked. Without parity, an additional stop bit is used to fill the character frame into a complete 11 bit asynchronous character. The maximum length of a Modbus RTU frame is 256 bytes, and the message frame format can be seen in Figure 3.

Figure 3: Modbus RTU Message Frame

When using Modbus ASCII transmission mode, the bus uses two ASCII characters to send an 8-bit byte in the message. When the physical communication link or device capability cannot meet the timing management requirements of RTU mode, this mode is used. Due to the requirement of sending two characters per byte, this mode is less efficient than RTU mode. The ASCII transmission mode occupies 10 bits per byte when sending frames, and its format is: 1 starting bit; 7 data bits, first send the least significant bit; 1 parity bit; 1 stop position. To ensure maximum compatibility with other products, it is recommended to also support the no checksum mode, which requires 2 stop bits when using no checksum to ensure complete 10 bit timing. In ASCII transmission mode, a message is distinguished by specific frame start and frame end characters. A message must start with a colon (:) character (in hexadecimal ASCII code 3A) and end with carriage return line break (CRLF) (in hexadecimal ASCII code OD and OA). The end character can be changed by specific Modbus application commands to modify the LF character (see GB/T19582.1-2008). The ASCII transmission mode message frame format can be seen in Figure 4.

Figure 4: Modbus ASCII Message Frame

Each data byte needs to be encoded with two characters. Therefore, in order to ensure compatibility between ASCII mode and RTU mode at the Modbus application level, the maximum data length of ASCII data fields (2x252) is twice the maximum data length of RTU data fields (252). Therefore, the maximum length of a Modbus ASCII frame is 513 characters.

- Modbus Serial Link Communication Security Service Definition and Description

At present, according to the definition in the standard GB/T 19582.2-2008 and IEC 61158 CPF15 (FDIS)-2006, for Modbus serial links based on EIA/TIA-485-A and EIA/TIA-232-E, the standard requires any EIA/TIA-485 multi-point system, whether in a 2-wire configuration or a 4-wire configuration, to have a maximum length of 1000m for cables with a maximum baud rate of 9600bit/s and AWG26 (or thicker) specifications, and also explicitly requires that for RS485 Modbus, a sufficiently wide cable diameter should allow for a maximum length of 1000m, and for RS485 Modbus using Category 5 cables, the maximum length can reach 600m, currently there is no related security standard for serial link communication scenarios. As an application level industrial communication protocol, with the evolution of more advanced attack technologies, it is very necessary and valuable to provide targeted security technology standards for different deployment scenarios to continuously improve the Modbus industrial application security guarantee system and ecosystem. For example, in the case of modern advanced hardware side channel attacks, especially in the presence of relay serial or bridged networks, transmitting Modbus industrial control protocol instructions and data in plaintext over a fully serial link medium can easily lead to data leakage or malicious modification by intermediaries, posing serious data or instruction security risks. Therefore, it is necessary to supplement, improve and enhance the intrinsic communication security of Modbus protocol under serial link mode, in order to adapt to various rich networking scenarios allowed in the related standards, for reference by relevant institutions and organizations in various industries, and to ensure the application security of various industrial control systems in actual operation.

Considering the various performance differences between master and slave devices in industrial networks, the implementation of the extended serial link secure communication function defined in this draft recommendation can be achieved by (1) directly integrating the extended and embedded security modules on the master and slave devices, or (2) adding separate extended secure external modules or secure proxy modules to the master and slave devices, as shown in Figure 5. The specific implementation form is not within the scope of this draft recommendation.

Figure 5: Implementation form of reference function for Modbus security

The Modbus application layer protocol and its standard function codes used on serial links are described in detail in the national standards Modbus Security Specification, GB/T 19582.1-2008 and GB/T 19582.2-2008. The function code field of the Modbus data unit is encoded with one byte, and the valid function code range is decimal 1-255 (with 128-255 reserved for exception response). When sending a message from the client to the server device, the function code field notifies the server of which operation to perform, where the function code '0' is invalid. In the standard Modbus Security Specification, GB/T 19582.1-2008, the following function codes and sub-codes are not included in the content of this standard. These function codes and sub codes are specially reserved in the format of function codes/sub codes, or only function codes, and all sub-codes (0-255) are reserved: 8/19, 8/21-65535, 9, 10, 13, 14, 41, 42, 90, 91, 125, 126, and 127. Please refer to Annex III of the standard GB/T 19582.1-2008. For the standard function codes defined in Modbus Security Specification, GB/T 19582.1-2008 and GB/T 19582.2-2008, this draft recommendation does not make any modifications to the function definitions. In addition, this draft recommendation does not occupy the valid business function code resources defined in the standard Modbus Security Specification, GB/T 19582.1-2008 to implement security service functions, while maintaining compatibility with the Modbus serial link protocol. This draft recommendation defines the function code "0" for the serial link secure communication service, which occupies an 8-bit byte and is used to define the content related to the extended Modbus serial link secure communication protocol. Among them, the original Modbus serial link protocol function codes and data fields are included as relevant parameters in the security protocol data to ensure that the corresponding function codes and data are confidential or integral during serial link communication.

In the secure mode of serial link communication, when using Modbus RTU transmission mode, the corresponding relationship between the original Modbus RTU frame and the Modbus secure RTU frame is shown in Figure 6, where the function code and data field of the original RTU frame are encrypted and included in protocol data field of the secure RTU frame:

Figure 6: Modbus Secure RTU Message Frame

In the secure mode of serial link communication, when using Modbus ASCII transmission mode, the corresponding relationship between the original Modbus ASCII frame and the Modbus secure ASCII frame is shown in Figure 7, where the function code and data field of the original ASCII frame are encrypted and included in protocol data field of the secure ASCII frame:

Figure 7: Modbus Secure ASCII Message Frame

- Modbus Serial Link Security Protocol Reference Specification

  - System workflow

The extended Modbus serial link security protocol specification defined in this draft recommendation aims to protect the transmission of industrial control system protocol parameters between master/slave stations (clients/servers) through the serial bus interface using the Modbus protocol, ensuring that protocol parameters are not maliciously modified or intercepted by intermediaries. On a Modbus serial link, the master station on the serial bus acts as the client and the slave station acts as the server. The specific workflow varies depending on the security protocols used. This draft recommendation provides multiple security protocols for industry reference and implementation.

1) Modbus serial link security protocol based on ECC public key certificate is mainly implemented through the following six processes, which will be described separately in the following chapters.

- Bidirectional public key authentication between client and server. When the client and server are bound for the first time, a mutual authentication protocol needs to be executed. Successfully executing the protocol can make the client and server confident in each other's reliability, and can also share a master key between both parties for secure processing of the protocol.

- Re-authentication of client and server. On the basis of the first binding and sharing of the master key, the mutual authentication protocol is generally not executed when booting up again, but the re authentication protocol is executed.

- SAC secure channel establishment. After successful authentication, the system enters the process of establishing the SAC secure channel. SAC secure channel is a secure channel established by the original Modbus protocol function code and data encryption and authentication key (referred to as content key).

- Content key establishment. On the SAC secure channel, the client and server can establish content keys for encrypting the original Modbus protocol function codes and data.

- Protocol function code and data encryption transmission. On the basis of establishing the content key, when sending a request, the client transmits the encrypted original Modbus protocol request function code and data to the server through Modbus secure RTU or ASCII message frames. The encrypted original Modbus protocol function code and data are decrypted and further processed on the server. When sending a response, the server transmits the encrypted original Modbus protocol response function code and data to the client through Modbus secure RTU or ASCII message frames. The encrypted original Modbus protocol function code and data are decrypted and further processed on the client.

- For Modbus broadcast protocol interaction, based on the SAC secure channel, when establishing a content key, the client directly sends the corresponding individual broadcast content key to all slave servers on the bus. The content key for Modbus communication is the same for all slave servers. When sending a broadcast request, the client uses the broadcast content key to transmit the encrypted original Modbus protocol broadcast request function code and data to the server through Modbus secure RTU or ASCII message frames. The encrypted original Modbus protocol function code and data are decrypted and further processed on the server. When sending a response, the server uses the broadcast content key to transmit the encrypted original Modbus protocol response function code and data to the client through Modbus secure RTU or ASCII message frames. The encrypted original Modbus protocol function code and data are decrypted and further processed on the client.

2) The Modbus serial link security protocol based on passwords or pre-shared keys mainly includes the following four processes, which will be described separately in the following chapters.

- Bidirectional authentication between client and server. When the client and server are bound for the first time, a mutual authentication protocol needs to be executed. Successfully executing the protocol can make the client and server confident in each other's reliability, and can also share a master key between both parties for secure processing of the protocol.

- Content encryption key establishment. The client and server establish two content encryption keys based on passwords or pre shared keys to encrypt and protect the original Modbus protocol function codes and data, one for broadcasting Modbus communication and the other for separate communication between the slave and master stations. The content key used for broadcasting Modbus communication is the same for all slave servers.

- Protocol function code and data encryption transmission. On the basis of establishing the content key, when sending a request, the client transmits the encrypted original Modbus protocol request function code and data to the server through Modbus secure RTU or ASCII message frames. The encrypted original Modbus protocol function code and data are decrypted and further processed on the server. When sending a response, the server transmits the encrypted original Modbus protocol response function code and data to the client through Modbus secure RTU or ASCII message frames. The encrypted original Modbus protocol function code and data are decrypted and further processed on the client.

- Regarding Modbus broadcast protocol interaction. When sending a broadcast request, the client uses the broadcast content key to transmit the encrypted original Modbus protocol broadcast request function code and data to the server through Modbus secure RTU or ASCII message frames. The encrypted original Modbus protocol function code and data are decrypted and further processed on the server. When sending a response, the server uses the broadcast content key to transmit the encrypted original Modbus protocol response function code and data to the client through Modbus secure RTU or ASCII message frames. The encrypted original Modbus protocol function code and data are decrypted and further processed on the client.

3) The Modbus serial link security protocol based on post quantum hybrid signature public key certificate mainly includes the following five processes, which will be described separately in the following chapters.

- Bidirectional authentication between client and server. When the client and server are first bound, a certificate based bidirectional authentication protocol needs to be executed. Successfully executing this protocol can make the client and server confident in each other's reliability, and can also share a master key between both parties, which can be used for protocol simplification and key derivation services.

- Re-authentication of client and server. On the basis of the first binding and sharing of the master key, the mutual authentication protocol is generally not executed when booting up again, but the re authentication protocol is executed.

- Content encryption key establishment. The client and server establish two content encryption keys based on post quantum encapsulation algorithm for encrypting and protecting the original Modbus protocol function codes and data, one for broadcasting Modbus communication and the other for separate communication between the slave and master stations. The content key used for broadcasting Modbus communication is the same for all slave servers.

- Protocol function code and data encryption transmission. On the basis of establishing the content key, when sending a request, the client transmits the encrypted original Modbus protocol request function code and data to the server through Modbus secure RTU or ASCII message frames. The encrypted original Modbus protocol function code and data are decrypted and further processed on the server. When sending a response, the server transmits the encrypted original Modbus protocol response function code and data to the client through Modbus secure RTU or ASCII message frames. The encrypted original Modbus protocol function code and data are decrypted and further processed on the client.

- Regarding Modbus broadcast protocol interaction. When sending a broadcast request, the client uses the broadcast content key to transmit the encrypted original Modbus protocol broadcast request function code and data to the server through Modbus secure RTU or ASCII message frames. The encrypted original Modbus protocol function code and data are decrypted and further processed on the server. When sending a response, the server uses the broadcast content key to transmit the encrypted original Modbus protocol response function code and data to the client through Modbus secure RTU or ASCII message frames. The encrypted original Modbus protocol function code and data are decrypted and further processed on the client.

  - Modbus Serial Link Security Protocol Based on ECC Public Key Certificate

    - Key hierarchy structure

The key management in the Modbus serial link security protocol based on ECC public key certificates is suitable for high-value and high security factories and devices. Its trust model can be found in Appendix V, and the key management scheme is divided into four levels: Credentials layer, Authentication layer, SAC secure channel layer, and Content key layer, as shown in Figure 8. Table 1 and 2 summarize the names, storage, and usage of the main parameters in the security protocol, where the requirement for random numbers generators can be found in Appendix IV.

The keys at the credentials layer mainly include ECC public cryptographic protocol parameters, private keys of the master client and slave server, and public key certificate chains. The public key certificate chain includes root certificates, brand certificates, and device certificates. The device certificate includes the device's ID number, ECC public key, and other attribute information of the certificate. The key data at this level is mainly used to execute authentication protocols. After device initialization, the certificate must not be changed or replaced. It is recommended to use mature OTP memory based on eFuse or Anti-Fuse technology, one-time programmable non-volatile memory, and data that cannot be reversibly modified after writing. The definition of protection methods for these data is not within the scope of this draft recommendation.

For the master client, the keys at the authentication layer are mainly DHSK and AKM; for the slave server, there are mainly two keys, namely DHSK and AKH. Among them, DHSK is a shared master key negotiated after a two-way authentication protocol, while AKM and AKH are exported by DHSK. These three keys are long-term keys that should be stored in the device's NVM and must be guaranteed not to be modified or intercepted by third parties. It is recommended to use a mature TEE environment, and the protection methods for these keys are not within the scope of this draft recommendation.

The SAC secure channel layer has two keys, SAK and SEK. The former is used for message authentication based on SAC, while the latter is used for message encryption based on SAC. The SAK and SEK keys are generated through the SAC initialization protocol executed by both parties and are temporary keys shared between the client and server. It should be stored in the NVM of the device and must ensure that it cannot be modified or accessed by third parties. It is recommended to use a mature TEE environment, and the definition of protection methods for SAK and SEK keys is not within the scope of this draft recommendation.

The content key layer first generates the content key CK and an initial vector CIV based on the SAC secure channel, while generating the keys BCK and BCIV required for broadcast communication. Then, the content key CK and initial vector CIV are used to encrypt and decrypt Modbus communication data using AES. The generation of CK key and vector CIV key is led by the master client, and the slave server obtains the same CK key and vector CIV through the SAC secure channel. Both are generated temporarily, and under certain conditions, the system must update the content key to provide high-strength security. Please refer to the following chapters for detailed instructions.

Table 1: Credential Parameters

| Parameter | Description | Volatile or non-volatile storage | Used for exchange or local use |
| --- | --- | --- | --- |
| Root certificate | Certificate of system trust root | Non volatile storage | Local use (non replaceable) |
| Brand certificate | Brand certificate of device manufacturer | Non volatile storage | Used for exchange (non replaceable) |
| Device certificate | Public key certificates for the main and slave devices under the brand | Non volatile storage | Used for exchange (non replaceable) |

Table 2: Main Parameters and Symbols of the System

| Parameter | Description | Volatile or non-volatile storage | Used for exchange or local use |
| --- | --- | --- | --- |
| EC_p | ECC Parameters | Non volatile storage | Local use (non replaceable) |
| EC_a | ECC Parameters | Non volatile storage | Local use (non replaceable) |
| EC_b | ECC Parameters | Non volatile storage | Local use (non replaceable) |
| EC_N | ECC Parameters | Non volatile storage | Local use (non replaceable) |
| EC_G | ECC Parameters | Non volatile storage | Local use (non replaceable) |
| MDP/HDP | Device Public Key | Non volatile storage | Used for exchange (non replaceable) |
| MDQ/HDQ | Device private key | Non volatile storage | Local use (non replaceable) |
| DHX/DHY | The power of DH modular exponentiation operation | volatile storage | Local use |
| DHPM/DHPH | The public key generated after DH modular exponentiation operation | volatile storage | exchange |
| DHSK | DH key (master key) | Non volatile storage | Local use |
| AKM | Master authentication key | Non volatile storage | Local use |
| AKH | Slave authentication key | Non volatile storage | exchange |
| Ns_M/Ns_H | Random numbers used for SAC channel establishment | volatile storage | exchange |
| SEK | SAC data encryption key | volatile storage | Local use |
| SAK | SAC Data Authentication Key | volatile storage | Local use |
| SIV | SAC initialization vector | Non volatile storage | Local use |
| Kp | Precursor used to calculate CK key | volatile storage | exchange |
| CK/BCK | Content encryption key for Modbus protocol function code and data | volatile storage | Local use |
| CIV/BCKIV | Content encryption initialization vector | volatile storage | Local use |

Figure 8: Key Hierarchy Structure

The overall key generation and usage process is as follows:

After power on start-up, the master and slave devices first perform a power on self-test. If it is normal, the master client should check if it can be re authenticated after power on; If the master client and slave server devices have been successfully bound before, re authentication is not necessary. Under the premise of successful binding, the two stages of certificate verification, DH key exchange, and authentication key verification can be skipped and directly enter the SAC channel establishment process. Otherwise, SAC authentication key generation for identity authentication must be performed first; After the SAC channel is established, the master and slave devices enter the content key establishment process. After the SAC channel and content key are established, the master and slave devices use content key encryption to protect the corresponding business protocol parameters during business operation. If for some reason, such as connection establishment failure or device busy, the protocol runs out of time and SAC or content key cannot be updated in a timely manner, corresponding errors can be prompted to device users or administrators on the industrial site.

    - Certificate and identity authentication mechanism

Authentication based on public key certificates includes certificate verification and DH key exchange, as well as verification of intermediate authentication keys. Firstly, the authentication process of the first stage master client and slave server begins by exchanging their respective certificate chains and DH public keys. The master client must verify the certificate chain of the slave server and the checksum generated by the slave server, and similarly, the slave server must verify the certificate chain of the master client and the checksum generated by the master client. During the verification process, the slave server will extract the CLIENT ID (64 bit data) from the master client's public key certificate, and similarly, the master client will also extract the SERVER ID (64 bit data) from the slave server's public key certificate.

Then, in the second stage, the data exchanged between the slave server and the master client using the authentication protocol in the first stage can obtain a long-term DH secret key and a long-term authentication key (This long-term authentication key can reduce the first stage of public key authentication operations when the master and slave devices are powered on for re authentication, as described in the "Authentication Context and Re-authentication" section later.). The authentication key is obtained by calculating the DH secret key and the IDs of both parties. Both parties should store the authentication key in NVM for future calculation of SAC secure channel encryption key and content key. Afterwards, the slave server will send a request to the master client to confirm the authentication key. Upon receiving the confirmation message, the slave server must compare the received key value with the stored key value. If the two match, it can prove that the master client is legitimate.

The steps of the device authentication protocol based on public key certificates are shown in Figure 9, and the message semantics are described in Table 3:

Figure 9: Authentication Message Exchange

Table 3: Authentication Message Exchange Instructions

| No. | Description |
| --- | --- |
| 1 | The slave server sends ss_open_req APDU to the master client. |
| 2 | The master client responds with ss_open_cnf APDU. |
| 3 | Send ss_data_req APDU from the slave server to the master client. The message contains: - Device certificate (Server_dev_Cert) of slave server; - Brand certificate (Server_Brand_Cert) of slave server; - ID of the requested data type. |
| 4 | The master client responds with ss_data_cnf APDU. The message contains: - DH public key (DHPH) of master client; - Device certificate for master Client (Client_Dev_Cert); - Brand certificate (Client_Brand_Cert) of master client. Failure to respond to this message may result in a system error, possibly due to the master client being unable to verify data from the slave server. |
| 5 | The slave server then sends the ss_data_req APDU. The message contains: - DH public key (DHPM) of slave server; - Verification value RM; - ID of the requested data type. Failure to respond to this message may result in a system error, possibly due to the slave server being unable to verify data from the master client. |
| 6 | The master client responds with ss_data_cnf APDU. The message contains: - A verification value RH. Failure to respond to this message resulted in a system error, possibly due to the inability of the master client to verify data from the slave server. |
| 7 | The slave server sens ss_data_req APDU to the master client. The message contains: - The data type ID of the requested nonce as Challenge. |
| 8 | The master client responds with ss_data_cnf APDU. The message contains an SM3(AKH, Challenge), Either it is valid or it is all zero, indicating illegality. Failure to respond to this message will result in a system error. |
| 9 | The slave server calculates and compares the response values received locally from the station. If there is inconsistency, it will result in a system error. |

1) The definition of ss_open_req APDU is as follows:

The slave server sends this APDU to inquire about the ID mask of the encryption system supported by the master client. The syntax is shown in Table 4.

Table 4: ss_open_req APDU syntax

| syntax | No. of bits | Mnemonic |
| --- | --- | --- |
| ss_open_req() { ss_open_req_tag length_field()=0 } | 24 | uimsbf |

ss_open_req_tag: 0x9F 90 01.

length_field: The length of the APDU payload, expressed in ASN. 1 BER format.

2) The definition of ss_open_cnf APDU is as follows:

The master client sends this APDU to the slave server, notifying them of the encryption system IDs supported by the slave server. The syntax is shown in Table 5.

Table 5 ss_open_cnf APDU syntax

| syntax | No. of bits | Mnemonic |
| --- | --- | --- |
| ss_open_cnf() { ss_open_cnf_tag length_field() ss_system_id_bitmask } | 24 8 | uimsbf bslbf |

ss_open_cnf_tag: 0x9F 90 02.

ss_system_id_bitmask: Each of the 8 bits indicates the encryption system version supported by a master client. The slave server can choose the highest version supported by both ends. The lowest bit indicates version 1, there is no version 0. The encrypted version defined in this section is 1. ss_system_id_bitmask lowest position 1 (indicating support for version 1).

3) The definition of ss_data_req APDU is as follows:

The slave server sends an ss_data.req message to the master client to transmit protocol related data, while requesting a response from the master client. The ss_data_req APDU syntax is shown in Table 6. ss_data_req APDU is used for data transmission that does not require authentication or encryption. Mainly used for authentication, authentication key verification, and SAC key calculation.

Table 6: ss_data_req APDU syntax

| syntax | No. of bits | Mnemonic |
| --- | --- | --- |
| ss_data_req() { ss_data_req_tag length_field() ss_system_id_bitmask send_datatype_nbr for (i=0; i<send_datatype_nbr; i++) { datatype_id datatype_length data_type } request_datatype_nbr for (i=0; i<request_datatype_nbr; i++) { datatype_id } } | 24 8 8 8 16 8*datatype_length 8 8 | uimsbf bslbf uimsbf uimsbf uimsbf bslbf uimsbf uimsbf |

ss_data_req_tag: 0x9F 90 03.

ss_system_id_bitmask: Each of the 8 bits indicates the encryption system version supported by a master client.

send_datatype_nbr: This message contains the number of data categories.

datatype_id: Data type ID, see Annex III.

datatype_1ength: The byte length of datatype shaped data.

data_type: The specific data represented by datatype_id.

request_datatype_nbr: The number of data categories that the master client should include in its response.

datatype_id: The data number that should be included in the response of the master  client, see Annex III.

4) The definition of ss_data_cnf APDU is as follows:

The master client sends an ss_data_cnf message to transmit protocol related data to the slave server. The ss_data_cnf APDU syntax is shown in Table 7. ss_data_cnf APDU is used for data that does not require authentication or encryption, mainly for authentication, authentication key verification, and SAC key calculation. For data that requires authentication or encryption, use ss_Sac_data_cnf APDU.

Table 7: ss_data_cnf APDU syntax

| syntax | No. of bits | Mnemonic |
| --- | --- | --- |
| ss_data_cnf() { ss_data_cnf_tag length_field() ss_system_id_bitmask send_datatype_nbr for (i=0; i<send_datatype_nbr; i++) { datatype_id datatype_length data_type } } | 24 8 8 8 16 8*datatype_length | uimsbf bslbf uimsbf uimsbf uimsbf bslbf |

ss_data_cnf_tag: 0x9F 90 04.

ss_system_id_bitmask: Each of the 8 bits indicates the encryption system version supported by a master client.

send_datatype_nbr: This message contains the number of data categories.

datatype_id: See Annex III.

datatype-length: The byte length of the data.

The specific data represented by data_type: datatype_id can be found in Annex III.

The protocol APDU format that includes specific data types in the authentication protocol is shown in Table 8 below:

Table 8: APDUs containing specific data type parameters in the authentication protocol

| Step | action | APDU | content |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | Send public key certificate chain from the slave server to the master client | ss_data_req | send _datatype_nbr=2 |  |  |  |  |  |  |  |
|  |  |  | i |  | datatype_id |  |  |  | datatype_len |  |
|  |  |  | 0 |  | 11(Server_Dev_Cert) |  |  |  | 1280 bits |  |
|  |  |  | 1 |  | 4(Server_Brand_Cert) |  |  |  | 1280 bits |  |
|  |  |  | request_datatype_nbr=3 |  |  |  |  |  |  |  |
|  |  |  | i |  | datatype_id |  |  |  |  |  |
|  |  |  | 0 |  | 8 (DHPH) |  |  |  |  |  |
|  |  |  | 1 |  | 10 (Client_Dev_Cert) |  |  |  |  |  |
|  |  |  | 2 |  | 3 (Client_Brand_Cert) |  |  |  |  |  |
| 2 | The master client sends DHPH and two public key certificates | ss_data_cnf | send_datatype_nbr=3 |  |  |  |  |  |  |  |
|  |  |  | i |  | datatype_id |  |  |  | datatype_len |  |
|  |  |  | 0 |  | 8(DHPH) |  |  |  | 512 bits |  |
|  |  |  | 1 |  | 10(Client_Dev_Cert) |  |  |  | 1280 bits |  |
|  |  |  | 2 |  | 3(Client_Brand_Cert) |  |  |  | 1280 bits |  |
| 3 | Sending DHPM and checksum RM from the slave server | ss_data_req | send_datatype_nbr=4 |  |  |  |  |  |  |  |
|  |  |  | i |  |  | datatype_id |  | datatype_len |  |  |
|  |  |  | 0 |  |  | 9(DHPM) |  | 512 bits |  |  |
|  |  |  | 1 |  |  | 12(RM) |  | 256 bits |  |  |
|  |  |  | request_datatype_nbr=1 |  |  |  |  |  |  |  |
|  |  |  | i | datatype_id |  |  |  |  |  |  |
|  |  |  | 0 | 20 (status_field) |  |  |  |  |  |  |
| 4 | The master client sends the verification value RH | ss_data_cnf | send_datatype_nbr=1 |  |  |  |  |  |  |  |
|  |  |  | i | datatype_id |  |  |  |  | datatype_len |  |
|  |  |  | 0 | 13(RH) |  |  |  |  | 256 bits |  |
| 5 | Request challenge from the slave server to the master client | ss_data_req | request_datatype_nbr=1 |  |  |  |  |  |  |
|  |  |  | i |  | datatype_id |  |  |  |  |
|  |  |  | 0 |  | 14(auth_nonce) |  |  |  |  |
| 6 | The master client sends response | ss_data_cnf | send_datatype_nbr=1 |  |  |  |  |  |  |
|  |  |  | i | datatype_id |  |  | datatype_len |  |  |
|  |  |  | 0 | 28(Challenge_response) |  |  | 256 bits |  |  |

Note: During the certificate authentication phase, APDU messages will encapsulate the Modbus serial link data unit field with a success number of 0. For APDUs larger than 252 bytes (the maximum data field length of Modbus RTU frames), such as ss_data_req APDUs, which exchange certificates with approximately 422 bytes, they can be divided into two Modbus serial link message frames for transmission. Since the transmission operation is strictly serial and has a sequence, the upper layer application can concatenate two or more segmented APDUs based on the tag (ss_data_cnf_tag) and length_field () fields of the protocol.

The authentication process of the slave server is shown in Figure 10.

Figure 10: Authentication Process of Slave Server

The description of authentication process of slave server in Figure 10 is as follows:

  - Before starting the authentication process from the server, all sessions should have been opened;

  - Send its own certificate chain from the slave server;

  - The slave server waits for a response message from the master client;

  - The slave server must verify the validity of the certificate responded by the master client. If the certificate is invalid, proceed to step 19);

  - The slave server must verify the validity of the DHPH value. If it is invalid, proceed to step 19);

  - Generate a random DHY from the slave server;

  - Calculate the DH public key DHPM from the slave server;

  - The slave server must check if the DHPM value is valid. If it is invalid, proceed to step 19);

  - Generate verification value RM from the slave server;

  - Sending protocol data from the slave server;

  - The slave server waits for the master client to send back a response message;

  - The slave server must check whether the RH of the response from the master client is correct. If not correct, proceed to step 19);

  - Calculate AKM from the slave server;

  - Request AKH from the slave server to the master client;

  - The slave server should receive AKH within 5 seconds. Otherwise, proceed to step 19);

  - The slave server must check if the received response contains a valid AKH. If the received AKH is invalid, proceed to step 19);

  - The slave server must check whether the received AKH is consistent with AKM. If there is inconsistency, proceed to step 19);

  - The authentication of the slave server has been successfully completed;

  - Send error message (optional);

  - Authentication failed.

Figure 11: Authentication process of the master client

The authentication process of the master client is shown in Figure 11.

- Session is successfully opened;

- The master client receives a certificate chain from the slave server;

- The master client must check if the received certificate is valid;

- The master client generates a random DHX;

- The master client calculates the DH public key DHPH;

- The master client must check whether the calculated DHPH is valid;

- The master client sends DHPH and master client certificate chain to the slave server;

- The master client is waiting for a response message from the slave server;

- The master client must verify that the DHPM received from the slave server is valid;

- The master client must verify whether the received slave server checksum is valid;

- The master client calculates the verification value RH;

- The master client sends RH to the slave server;

- The master client calculates AKH;

- Authentication successful;

- Any error will result in authentication failure;

- The master client receives a request from the slave server;

- The master client responds with AKH value;

Note: The slave server can repeatedly send requests to AKH until the master client responds.

- Authentication completed.

The calculations involved in the certificate based authentication process are shown in Figure 12, with specific Descriptions provided in Table 9.

Figure 12: Calculation steps involved in the authentication process

Table 9: Description of Calculations Involved in the Certification Process

| No. | Description |
| --- | --- |
| 1) | The slave server sends its public key certificate chain to the master client |
| 2) | The master client verifies the validity of the two received certificates from the slave servers. |
| 3) | The master client uses an appropriate PRNG to generate a random DH exponent x (DHX). |
| 4) | The master client calculates its own DH public key (DHPH). |
| 5) | The master client sends the DHPH, master client device certificate, and master client device brand certificate to the slave server. |
| 6) | The slave server verifies the validity of the two certificates received from the master client. |
| 7) | The slave server generates a random DH exponent y (DHY) using an appropriate PRNG. |
| 8) | The slave server computes its own DH public key (DHPM) . |
| 9) | The slave server calculates the verification value RM as follows: a) Firstly, use the inverse element MDQ-1 of its private key to calculate xG = E_M(MDQ-1, DHPH); b) Calculate and store DHSK= E_M(y, xG); c) Calculate RM=SM3(xG¦¦DHPM¦¦DHSK¦¦SERVER ID¦¦ss_system_id_bitmask ¦¦mode); Note: E.M (*, *) represents scalar multiplication on an elliptic curve, where the first parameter is an element in a finite field and the second parameter is a point on the elliptic curve. |
| 10) | The slave server sends DHPM and RM to the master client. |
| 11) | The master client verifies the received parameters as follows: a) Firstly, use the inverse element HDQ-1 of the private key to calculate yG = E_M(HDQ-1, DHPM); b) Calculate and store DHSK = E_M(x, yG); c) Calculate RM '= SM3(xG¦¦DHPM¦¦DHSK¦¦SERVER_ID¦¦ss_system_id_bitmask ¦¦mode); d) Verify whether the received RM is consistent with the calculated RM '. Note: E.M (*, *) represents scalar multiplication on an elliptic curve, where the first parameter is an element in a finite field and the second parameter is a point on the elliptic curve. |
| 12) | The master client uses the data calculated in steps 4) and 11) to calculate the verification value RH: RH = SM3(yG¦¦DHPH¦¦DHSK¦¦CLIENT_ID¦¦ss_system_id_bitmask¦¦mode). |
| 13) | The master client sends RH to the slave server. |
| 14) | The slave server verifies the received parameters using the data calculated in steps 7) and 9): a) Calculate RH '= SM3(yG¦¦DHPH¦¦DHSK¦¦CLIENT_ID¦¦ss_system_id_bitmask¦¦mode); b) Verify whether the received RH is consistent with the calculated RH '. The slave server calculates and stores the authentication key (AKM). |
| 15) | The master client calculates and stores the authentication key (AKH). |
| 16) | The slave server start to enter the second stage of the authentication process: request the current authentication key AKH from the master client through the response APDU. |
| 17) | The master client uses the corresponding APDU to send AKH to the slave server. |
| 18) | The slave server compares the received AKH with its own calculated AKM. If they are different, it will result in a system error. |

The DH public keys (DHPH and DHPM) are temporary data that should be cleared and deleted after the authentication protocol is completed.

The DH public key is calculated as follows:

DHPH＝E_M(x, MDP)，DHPM＝E_M(y, HDP)

Note that in the above equation:

The values x (DHX) and y (DHY) are generated by the built-in PRNG of the device. The values of x (DHX) and y (DHY) should be kept secret locally and cleared and deleted after the authentication protocol ends. In order to generate secure protocol parameters, after the master client calculates DHPH, the following checks must be performed:

DHPH is neither an infinite point nor a generator G, and the value of E-M (EC-N, DHPH) is an infinite point.

Similarly, after calculating DHPM from the station server, the following checks must be performed:

DHPM is neither an infinite point nor a generator G, and the value of E-M (EC-N, DHPM) is an infinite point.

The authentication keys (AKH and AKM) can be used in the calculation process of SAC keys and CK. The authentication key is generated when the master client and slave server are first bound and stored in NVM, which can be used for repeated authentication in the future. The calculation process is as follows:

AKM = AKH = HKDF(DHSK, SERVER_ID,CLIENT_ID,)

Note that this recommendation stipulates that HKDF uses the HAMC function HMAC-SM3 based on SM3, or other private HASH based functions if possible. The first parameter is the key, followed by the data. If there are multiple data parameters, all data should be concatenated, the same applies below.

The relevant parameter columns are shown in Table 10.

Table 10: Parameters in Authentication Key Calculation

| Parameter | No. of bits | description |
| --- | --- | --- |
| DHSK | 512 | The DH secret key shared during the authentication process. |
| CLIENT_ID | 64 | Included in the public key certificate of the master client device. |
| SERVER_ID | 64 | Included in the public key certificate of the slaver server device. |

Authentication context and re-authentication: Authentication context is a secure data object stored in NVM with a validity period, and its storage format is not strictly defined in this draft recommendation. After the expiration date, it is generally considered that the context is not trustworthy and needs to be deleted and a new binding relationship needs to be established in the future. However, in the implementation of actual products, it should be given sufficient security to ensure that it is not changed by attackers. The authentication context is used to store the results of public key authentication between both parties of the device, and is used to perform the re authentication process. The information that should be included in the authentication context of the slave server and master client is shown in Table 11.

Table 11: Information in Authentication Context

| The information that should be included in the authentication context |  |
| --- | --- |
| Slave server | Master client |
| CLIENT_ID DHSK AKM Encryption mode | SERVER_ID DHSK AKH Encryption mode |

After the master client and slave server are first bound, the device should associate the DHSK and authentication key negotiated during the public key authentication process with an authentication context. The master client should support at least 247 authentication contexts, and the slave server should support at least 1 authentication context.

After establishing a session, the master client and slave server can first perform an authentication key verification step to check if there is an existing binding relationship between the two devices. If there is, there is no need to perform a public key authentication protocol. If the slave server has a valid authentication context, it should directly request the master client to send an AKH and then check if the received AKH matches the AKM in the local authentication context. If there is inconsistency, the slave server shall attempt up to 246 more times (as the master client may have 247 authentication contexts). When there are no other available authentication contexts for the master client, the master client should send a zero value as a response to the AKH request from the slave server, and the slave server should start executing the public key authentication protocol. If the master client has the authentication context of the slave, it should first send the AKH in the context of the slave. If the AKH is inconsistent with the AKM of the slave server, the slave server should attempt to request AKH again, and the master client should send other AKHs in other authentication contexts. So, re-powering on the slave server may not necessarily execute the public key authentication protocol to reduce authentication time. But when the slave server accesses an unbound master client, or when the master client accesses an unbound slave server, the identity authentication protocol related to the public key certificate is required to be executed.

    - Establishing secure authentication channel Mechanism

After authentication is completed, the protocol will establish a secure authentication channel (SAC), which is mainly used for content key establishment and transmission. Figure 13 is a schematic diagram of its function.

Figure 13: Schematic diagram of SAC function

Figure 14 shows the workflow of SAC channel establishment, and Table 12 provides a description of the process in Figure 14.

Figure 14: SAC channel establishment process

Table 12: Process Description for SAC Channel Establishment

| No. | Description |
| --- | --- |
| 1),2) | Authentication protocol. The slave server and master client must have successfully executed the mutual authentication protocol. |
| 3)～6) | SAC initialization. Both the slave server and the master client need to first perform SAC initialization to generate key data and set the initial state of SAC. |
| 7),8) | SAC synchronization request and confirmation. If the slave server has correctly initialized SAC, the slave server should send an APDU requesting synchronization to the master client. After the master client successfully responds, both parties can start using SAC. |
| 9)～15) | Generate and send SAC messages. Generate SAC messages by adding message headers and authentication domains to the payload and encrypting them (optional). |
| 16)～21) | Receive and verify SAC messages. Validate the received SAC message and proceed with further processing under the condition that the message is valid. Note: SAC can be used for bidirectional sending and receiving of messages; |

The SAC initialization process can refer to Figure 15, and Table 13 provides specific steps for Description.

Figure 15: SAC initialization process

Table 13: Initialization steps of SAC

| No. | Description |
| --- | --- |
| 1) | When the slave server detects that the conditions are met, it initiates the SAC initialization process. |
| 2) | The slave server generates a random number Ns_S, which is used as part of the data involved in SAC key calculation. |
| 3) | The slave server sends the ss_data_req APDU to the master client, including the following parameters: - Ns_M; - SERVER_ID. |
| 4) | The master client generates a random number Ns_H, which is used as part of the data for SAC key calculation. |
| 5) | After receiving the ss_data_req APDU from the slave server, the master client should respond to the slave server with the ss_data_cnf APDU, which includes the following parameters: - Ns_H; - CLIENT_ID. Failure to respond to ss_data_cnf message will result in a system error |
| 6) | The slave server checks whether the received CLIENT_ID matches the pre-stored one. If they match, start calculating SAK and SEK and set or reset SAC status. If they do not match, it will cause a system error. Note: The pre-stored SERVER_ID/CLIENT_ID is in the "authentication context". |
| 7) | The master client checks whether the received SERVER_ID matches the pre-stored one. If they match, start calculating SAK and SEK and set or reset SAC status. If they do not match, it will cause a system error. Note: The pre-stored SERVER_ID/CLIENT_ID is in the "authentication context". |
| 8) | The slave server sends ss_stync_req APDU to the master client to indicate that the master client SAK needs to be refreshed. When the slave server has completed initialization, it will send a synchronization request indicating that the slave server is ready to use SAC. |
| 9) | The master client sends ss_sync_cnf APDU to the slave server as a confirmation response, indicating that the master client is also ready to use SAC. Failure to respond to ss_sync_cnf messages will result in a system error. |

The SAC initialization protocol is initiated by the slave server and passively responded by the master client. The re-initialization process is optional. This protocol is triggered by the following conditions:

- Power on restart; After system start-up or restart and there is a valid AKM in NVM.

- Certification; When there is no valid AKM in NVM, the system will enter the re authentication protocol, and after the authentication protocol is completed, trigger the SAC initialization protocol.

- Message counter overflow.

- System allows SAC to reinitialize.

APDU definition used during SAC initialization process:

The definitions of ss_open_req and ss_open_cnf APDU are the same as those in the authentication mechanism in the previous section.

- ss_sync_req APDU

Used for SAC key calculation, the slave server sends the APDU after the key calculation is completed to indicate that it is ready to use the newly calculated key. The syntax of ss_sync_req APDU is shown in Table 14.

Table 14: ss_sync_req APDU syntax

| syntax | No. of bits | Mnemonic |
| --- | --- | --- |
| ss_sync_req() { ss_sync_req_tag length_field()=0 } | 24 | uimsbf |

ss_sync_req_tag:0x9F 90 05.

2）ss_sync_cnf APDU

This APDU is the response of the master client to the ss_sync_req APDU, indicating that the master client has completed the key calculation. The syntax of the ss_sync_cnf APDU is shown in Table 15.

Table 15: ss_sync_cnf APDU syntax

| syntax | No. of bits | Mnemonic |
| --- | --- | --- |
| ss_sync_cnf() { ss_sync_cnf_tag length_field()=1 status_field } | 24 8 | uimsbf uimsbf |

ss_sync_cnf_tag: 0x9F 90 06.

status_field: This byte returns the status of the master client. Table 16 lists possible values.

Table 16: Possible values of status_field

| status_field | value |
| --- | --- |
| OK No resource Busy Authentication failure Reserved | 00 01 02 03 04-FF |

The APDU format of the SAC initialization protocol, which includes specific data types, is shown in Table 17 below:

Table 17: APDUs containing specific data types in SAC initialization

| step | action | APDU | content |  |  |
| --- | --- | --- | --- | --- | --- |
| 1 | The slave server sends SERVER_ID and a random number | ss_data_req | send _datatype_nbr =2 |  |  |
|  |  |  | i | datatype_id | datatype_len |
|  |  |  | 0 | 2(SERVER_ID) | 64 bits |
|  |  |  | 1 | 16(Ns_M) | 64 bits |
|  |  |  | Request_datatype_nbr=2 |  |  |
|  |  |  | i | datatype_id |  |
|  |  |  | 0 | 1(CLIENT_ID) |  |
|  |  |  | 1 | 15(Ns_H) |  |
| 2 | The master client responds with CLIENT_ID and a random number | ss_data_cnf | send_datatype_nbr=2 |  |  |
|  |  |  | i | datatype_id | datatype_len |
|  |  |  | 0 | 1(CLIENT_ID) | 64 bits |
|  |  |  | 1 | 15(Ns_H) | 64 bits |
| 3 | The slave server informs the master client that it has completed the SAC key calculation | ss_sync_req |  |  |  |
| 4 | The master client informs the slave server that the SAC key calculation has been completed | ss_sync_cnf | status_field |  |  |

Note: During the secure channel establishment phase, APDU messages will encapsulate the Modbus serial link data unit field with a success code of 0, which is much smaller than 252 bytes (the maximum data field length of a Modbus RTU frame) of APDU. One Modbus serial link message frame can complete the interaction of APDU.

Figure 16 explains the operation process of the master client during SAC initialization, while Figure 17 explains the operation process of the slave server.

Note: If the condition in step 6 (or step 9) of Figure 17 is not met more than 3 times, it will result in a system error.

Figure 16: SAC initialization on master station client process

Figure 17: SAC initialization on slave server process

Calculating the authentication key SAK and encryption key SEK for SAC requires two steps:

- Calculation of key seed Ks.

- Export SAK and SEK.

The key seed Ks has a length of 256 bits. The slave server and master client should calculate the value of the key seed, and the master client must use the following method to calculate Ks_client:

Ks_client = HKDF(DHSK, AKH, Ns_H, Ns_M)

The slave server must calculate Ks_server using the following method:

Ks_server = HKDF(DHSK, AKM, Ns_H, Ns_M)

Under normal circumstances, Ks_server = Ks_client, this two keys herein are named Ks, with the parameters shown in Table 18.

The SAK and SEK exports are calculated as follows:

SEK=MSB128(Ks)，SAK=LSB128(Ks)

Table 18: Parameters in SAC Key Calculation

| parameter | No. of bits |
| --- | --- |
| DHSK | 512 |
| AKH/AKM | 256 |
| Ns_H | 64 |
| Ns_M | 64 |

Figure 18 explains the recommended process and related error handling for receiving messages from the site server SAC.

Figure 18: SAC Error Handling process

The SAC message format and semantic description are as follows:

The data messages transmitted as SAC payloads should be encapsulated in SAC messages, with the encapsulation format shown in Figure 19. The specific definition of SAC message syntax is shown in Table 19.

Figure 19: SAC packaging payload

Table 19: SAC message syntax

| syntax | No. of bits | Mnemonic |
| --- | --- | --- |
| message() { message_counter /* Message header start */ protocol_version authentication_cipher_flag payload_encryption_flag encryption_cipher_flag reserved_for_future_use length_payload /* Message header end */ /* Message body start */ if (payload_encryption_flag == MSG_FLAG_TRUE) { encrypted_payload } else if (payload_encryption_flag == MSG_FLAG_FALSE) { payload authentication } /* Message body end */ } | 32 4 3 1 3 5 16 length_payload * 8 + 128 length_payload * 8 128 | uimsbf uimsbf uimsbf bslbf uimsbf bslbf uimsbf bslbf bslbf bslbf |
| Note：MSG_FLAG_FALSE = 0，MSG_FLAG_TRUE = 1. |  |  |

Encoding and semantics of fields in SAC messages:

message_comounter: Data messages require a count value to identify the number of messages that have been transmitted.

protocol version: In this section, set to 0x0.

authentication_cipher_flag: This parameter specifies the algorithm that generates the authentication code, as shown in Table 20.

Table 20: Allowable values for authentication_cipher_flag

| content | alg. | description |
| --- | --- | --- |
| 0x0 | AES-128-XCBC-MAC | RFC 3566 |
| 0x1..0x7 | Reserved |  |
| - Note: Devices that comply with this version recognize "0x0" and ignore other unsupported values. - Note: There is an exception where the 128 bit output of MAC is used entirely as message authentication code in this section without clipping. |  |  |

payload_decryption-flag: This parameter indicates whether the payload is encrypted. '1' indicates encryption, '0' indicates no encryption. Devices that comply with this section are described as "1" and other unsupported values are ignored.

encryption_cipher_flag: This parameter indicates the encryption algorithm, as shown in Table 21.

Table 21: Allowable values for encryption_cipher_flag

| content | alg. | description |
| --- | --- | --- |
| 0x0 | AES-128, CBC mode | FIPS 197 |
| 0x1...0x7 | Reserved |  |
| Note: Devices that comply with this version recognize "0x0" and ignore other unsupported values. |  |  |

length_payload: This parameter represents the number of bytes in the payload, including padding data but not the number of bytes in the authentication code.

encrypted_payload: This field consists of encrypted data, including ciphertext for message payload, padding data, and authentication code.

payload: This field is composed of unencrypted data.

authentication: This field carries the authentication code for the data, which can be encrypted according to the indication of "payload_encryption_flag".

The data message submitted for SAC transmission when sending SAC messages should be processed according to the following steps:

  - Check if 'message_counter' overflows and update 'message_counter'.

  - Calculate the message authentication code.

  - If encryption is required, concatenate the authentication code and payload to encrypt the message (optional).

  - Construct SAC messages.

  - Transmitting messages.

If any step of the above process fails, the message and status (such as key set, counter, etc.) should be destroyed and an error generated.

The authentication code for SAC data messages is calculated as follows:

authentication = MAC{SAK}(length(header_hi)||i||header_hi||payload_pi)

Where:

MAC - The algorithm is identified by "authentication_cipher_flag";

SAK - is a 128 bit SAC authentication key;

Authentication code calculation is specific to the entire message, with parameters shown in Table 22.

Table 22: Parameters in MAC Calculation

| Parameter | No. of bits | Mnemonic |
| --- | --- | --- |
| length_hi | 8 | uimsbf |
| i | 32 | uimsbf |
| header_hi | length_hi * 8 | bslbf |
| payload_pi | length_payload * 8 | bslbf |

The parameter description is as follows:

  - i: This field is the 'message_counter' in the data message.

  - length_hi: This field is the number of bytes in the message header.

  - header_hi: This field represents the message header.

  - payload_pi: This domain carries the message payload. The original unencrypted payload.

If SAC messages need to be encrypted, the following method should be used:

encrypted_payloadi = E{SEK, SIV}(payload_pi||authentication_ai)

Where:

E - The encryption algorithm indicated by encryption_cipher_flag;

SEK -128 bit encryption key;

SIV - a fixed constant with a value of 0xB27097DEAF305D8A94C871D89525C7A0;

authentication_ai - This value is the authentication code MAC.

If the length of payload_pi is not an integer multiple of 128 bits, it should be padded with a "1" first, followed by several "0s" so that the padded result is an integer multiple of 128 bits. If the payload is not encrypted, it will not be filled.

SAC message reception should be processed according to the following steps:

  - Firstly, check if the message contains the correct "message_counter" and "protocol-version";

  - If 'payload_encryption-flag'=1, decrypt the message payload;

  - Calculate the message authentication code and verify the integrity of the message.

If any step of the above process fails, the message and status (such as the key set, counter, etc.) should be destroyed and an error should be generated.

Both the slave server and the master client should maintain a message counter (message_counter) locally to identify the number of messages that have been transmitted. The initial value of this counter is 0x01 (not 0x00), and every time a SAC message is sent or received, the counter value is incremented by 1. In the current counter value state, if a message needs to be sent, assign the current value of the counter to the "message_counter" field in the SAC message being sent. After the message is sent, increment the counter value by 1. In the current counter value state, if a message is received, the value of the "message_counter" field in the received SAC message should be the same as the current value of the counter. After receiving the message, the counter value needs to be incremented by 1. The receiving device should not receive messages with incorrect counter values. Incorrect counter values will result in a 'message sequence error'. When the value of the message sending counter of the slave server overflows (reaching 232), the slave server should stop using the current SAC key and start negotiating a new SAC key.

The decryption process of SAC data messages is as follows:

payload_pi || authentication_ai = D{ SEK, SIV}( encrypted_payloadi)

Where:

D - the algorithm indicated by "encryption_cipher_flag";

SEK -128 bit key;

SIV - a fixed constant with a value of 0xB27097DEAF305D8A94C871D89525C7A0;

Among them, 'authentication_ai' should be separated from 'payload_pi'. If 'payload_pi' is not an integer multiple of the 128 bit packet, then the message is obtained by padding a '1' and several '0s'. If the payload is not encrypted, there is no need to fill in any data.

The SAC data message contains an authentication domain, and its verification process is as follows:

authentication_ai’ = MAC{SAK}(length(header_hi)||i||header_hi||payload_pi)

Where:

MAC - The algorithm is identified by "authentication_cipher_flag";

SAK -128 bit SAC authentication key.

If the calculated 'authentication_ai' is not equal to the 'authentication_ai' exported from the message, the received message must be deleted and a 'message validation error' generated.

Note: The design of SAC channels can be used for various purposes, mainly for negotiating and updating content keys. The way SAC messages are encapsulated into Modbus serial link data unit fields with function code 0 is shown in Figure 20. Table 23 provides a description of the packaging steps. Since SAC is mainly used to synchronize and update CK content key data in this draft recommendation, as detailed in the next chapter, one Modbus serial link data unit can be sent completely in one message.

Figure 20: SAC message encapsulation

Table 23: SAC Message Encapsulation

| step | description |
| --- | --- |
| 1 | The system collects data objects to form SAC payloads |
| 2 | The system calculates message authentication code |
| 3 | The SAC payload and authentication code are encrypted. Add SAC header, APDU tag, and APDU length field before the encrypted SAC data |

    - Content Key Update Mechanism

The process of the content key CK update protocol is shown in Figure 21, and the description of CK Content Key Update Process is shown in Table 24.

Figure 22: CK Content Key Update Process

Table 24: Description of CK Content Key Update Process

| step | description |
| --- | --- |
| 1) | When the slave server detects the need for CK update, the slave server begins the CK update process. |
| 2) | The slave server generates a 256 bit nonce and calculates Kp=SM3 (nonce). |
| 3) | The slave server can immediately start calculating CIV and CK. |
| 4) | The slave server sends an ss_sac_data_req APDU containing the following parameters： - Kp； - SERVER_ID； |
| 5) | The master client checks that the received SERVER_ID matches the one in "authentication context". If they are consistent, the master client starts calculating CIV and CK, BCIV and BCK; If there is inconsistency, it will result in a system error. Note: The pre stored SERVER_ID/CLIENT_ID is in the "authentication context". |
| 6) | The master client generates a 256 bit nonce and calculates Kp_client = SM3 (nonce). Respond to ab ss_sac_data_cnf APDU containing CLIENT_ID and Kp_client. Failure to respond to this message will result in a system error. |
| 7) | The slave server checks that the received CLIENT_ID matches the one in "authentication context". If they are consistent, the slave server starts calculating CIV and CK; If there is inconsistency, it will result in a system error. Note: The pre stored SERVER_ID/CLIENT_ID is in the "authentication context". |
| 8) | The master client calculates CIV and CK, while the slave server calculates BCIV and BCK. |
| 9) | The slave server sends an ss_sac_sync_req APDU to the master client to indicate content key updates. When the slave server completes the initialization of the encryption process, it sends a synchronization request to the master client, informing them that the slave server is ready to use the new CK and BCK. |
| 10) | The master client responds with an ss_stac_sync_cnf APDU and confirms to the slave server that the master client is also ready to use the new CK and BCK. Failure to respond to this message will result in a system error. |

The update of content key CK is mainly initiated by the slave server, and the master client only passively responds. This process is optional, and the content key CK update protocol is triggered by any of the following conditions:

  - After successful completion of authentication and SAC initialization.

  - Periodic triggering (maximum key lifecycle parameter, configured by administrator).

  - The block counter has reached its upper limit.

  - Every time it restarts.

  - System allows SAC to reinitialize.

APDU definition used in the content key update protocol process:

1) ss_sac_data_req APDU

The slave server uses this APDU to send protocol specific data to the master client, while requesting a response. The syntax of ss_sac_data_req APDU is shown in Table 25. Unlike ss_data_req APDU, the data contained in this message is authenticated and encrypted. SAC uses the input data in Table 17 as the payload in the SAC message.

Table 25: ss_stac_data_req APDU syntax

| syntax | No. of bits | Mnemonic |
| --- | --- | --- |
| ss_sac_data_req() { ss_sac_data_req_tag length_field() sac_message() } | 24 | uimsbf |

ss_sac_data_req_tag: 0x9F 90 07.

sac_message: The format of this message was defined in the previous section, as shown in Figure 19 and Table 23.

payload_encryption-flag should be 1.

The definition of the payload of SAC messages is in Table 26.

Table 26: ss_sac_data_req Payload

| syntax | No. of bits | Mnemonic |
| --- | --- | --- |
| ss_system_id_bitmask send_datatype_nbr for (i=0; i<send_datatype_nbr; i++) { datatype_id datatype_length data_type } request_datatype_nbr for (i=0; i<request_datatype_nbr; i++) { datatype_id } | 8 8 8 16 8*datatype_length 8 8 | bslbf uimsbf uimsbf uimsbf bslbf uimsbf uimsbf |

ss_system_id_bitmask: Same as above.

send_datatype_nbr: This message contains the number of data categories.

datatype_id: See Annex III.

datatype_length: The byte length of the data in the form of a data type.

data_type: The specific data represented by datatype_id.

request_datatype_nbr: The number of data categories that the primary client should include in its response.

datatype_id: See Annex III.

2) ss_sac_data_cnf APDU

When data needs to be authenticated and encrypted, the master client should use this APDU to send protocol specific data to the slave server. The syntax of ss_sac_data_cnf APDU is shown in Table 27. The input data in SAC encapsulation Table 28 is used as the payload in the SAC message.

Table 27: ss_sac_data_cnf APDU syntax

| syntax | No. of bits | Mnemonic |
| --- | --- | --- |
| ss_sac_data_cnf() { ss_sac_data_cnf_tag length_field() sac_message() } | 24 | uimsbf |

ss_sac_data_cnf_tag: 0x9F 90 08.

sac_message: The format of this message is defined as above.

payload_encryption-flag should be 1.

The payload of SAC messages is defined in Table 28.

Table 28: ss_sac_data_cnf Payload

| syntax | No. of bits | Mnemonic |
| --- | --- | --- |
| ss_system_id_bitmask send_datatype_nbr for (i=0; i<send_datatype_nbr; i++) { datatype_id datatype_length data_type } | 8 8 8 16 8*datatype_length | bslbf uimsbf uimsbf uimsbf bslbf |

ss_system_id_bitmask: Same as above.

send_datatype_nbr: This message contains the number of data categories.

data_type_id: See Annex III.

datatype_length: The byte length of the data.

data_type: The specific data represented by datatype_id.

3) ss_sac_sync_req APDU

This APDU is used in the CK calculation process. The slave server sends this message to indicate that it has completed the new CK calculation. The syntax of ss_sac_sync_req APDU is shown in Table 29.

Table 29: ss_stac_sync_req APDU syntax

| syntax | No. of bits | Mnemonic |
| --- | --- | --- |
| ss_sac_sync_req() { ss_sac_sync_req_tag length_field() sac_message() } | 24 | uimsbf |

ss_sac_sync_req_tag: 0x9F 90 09.

sac_message: Same as above.

payload_encryption_flag should be 1.

The payload of the SAC message here is empty.

4) ss_sac_sync_cnf APDU

This APDU is used in the CK calculation process. The master client responds with this message to the ss_sac_sync_req APDU message sent from the slave server. The syntax of the ss_sac_sync_cnf APDU is shown in Table 30.

Table 30: ss_sac_sync_cnf APDU syntax

| syntax | No. of bits | Mnemonic |
| --- | --- | --- |
| ss_sac_sync_cnf() { ss_sac_sync_cnf_tag length_field() sac_message() } | 24 8 | uimsbf uimsbf |

ss_sac_cnf_req_tag: 0x9F 90 10.

sac_cessage: Same as above.

payload_encryption-flag should be 1.

The payload of this SAC message is a state domain. All possible values in the state domain are listed in Table 31.

Table 31: State Fields in ss_sac_sync_cnf

| Status_field | Value |
| --- | --- |
| OK No resource Master client busy Not required Reserved | 00 01 02 03 04-FF |

The APDU format of the protocol that includes specific data types in the content key update process is shown in Table 32 below:

Table 32: APDUs with specific data types included in the content key update process

| step | action | APDU | content |  |  |
| --- | --- | --- | --- | --- | --- |
| 1 | Send SERVER_ID and a random number from the slave server | ss_sac_data_req | send _datatype_nbr=3 |  |  |
|  |  |  | i | datatype_id | datatype_len |
|  |  |  | 0 | 2(SERVER_ID) | 64 bits |
|  |  |  | 1 | 7(Kp) | 256 bits |
|  |  |  | request_datatype_nbr=2 |  |  |
|  |  |  | i | datatype_id |  |
|  |  |  | 0 | 1(CLIENT_ID) |  |
|  |  |  | 1 | 20(Status_field) |  |
| 2 | The master client responds with CLIENT_ID and status information | ss_sac_data_cnf | send_datatype_nbr=3 |  |  |
|  |  |  | i | datatype_id | datatype_len |
|  |  |  | 0 | 1(CLIENT_ID) | 64 bits |
|  |  |  | 1 | 7(Kp_client) | 256 bits |
|  |  |  | 2 | 20(Status_field) | 8 bits |
| 3 | The slave server notifies the master client that the new CK calculation has been completed | ss_sac_sync_req |  |  |  |
| 4 | The master client responds to its own status information | ss_sac_sync_cnf | status_field |  |  |

Figure 22 shows the CK update operation on the slave server side, and the broadcast content key BCK can be generated along with it.

Figure 22: CK content key update process on the slave server side

Note: The key update timer is used to record the calculation time of the new CK.

Note: The maximum key lifecycle value is controlled by the administrator configuration system and is not within the scope specified in this draft recommendation. The decrease of this value is controlled by the slave server and only decreases when encrypting or decrypting content. For ease of processing, it is generally defined by the number of messages. So when CK is not in use, this value is not updated.

Note: The default upper limit of the block counter is 232, and the message may be divided into multiple block frames. The CK block counter limit is the number of processed crypto blocks since CK refresh. The number of block limit defined here is configured by the system and is not within specified in this draft recommendation.

Note: The initial key lifecycle refers to the first key lifecycle after SAC initialization.

Figure 23 shows the CK update operation of the master client.

Figure 23: CK content key update process of the master client

The calculation process of content key CK and broadcast content key BCK should be completed as follows:

The encryption algorithm requires CK and CIV, and the calculation of these two values requires two steps: key seed calculation and CK and CIV export.

Step 1: Calculate the key seed.

Kp = SM3(nonce)

Step 2: Export CK and CIV.

CK = MSB128(HKDF(Kp, SERVER_ID)), CIV = LSB128(HKDF(Kp, SERVER_ID))

Step 3: Export BCK and CIV for broadcast communication between the master client and all slave servers.

BCK = MSB128(HKDF(Kp_client, CLIENT_ID)),

BCIV = LSB128(HKDF(Kp_client, CLIENT_ID))

Where:

SERVER_ID represents the ID of the slave server, while CLIENT_ID represents the ID of the master client. CIV remains unchanged throughout each key lifecycle. CIV and CK change simultaneously. The current CIV is reused at the beginning of encrypting (decrypting) each Modbus data frame. During broadcast communication, BCK and BCIV should be changed simultaneously.

Note: When the aforementioned update CK operation is initiated by the slave station, if the master station does not update the corresponding Kc_client, the accompanying BCK will not change to prevent inconsistent BCK received by all other slave stations when a single slave station updates CK.

Generally, after power on, the BCK and BCIV of all slave servers can remain unchanged, or they can be updated by the master station through the SAC content key update protocol. The process of updating BCK protocol is shown in Table 33 separately.

Table 33: Description of the process of updating BCK protocol separately

| step | description |
| --- | --- |
| 1) | The master client generates a 256 bit nonce and calculates Kp_client=SM3 (nonce). |
| 2) | The master client can immediately start calculating BCIV and BCK. |
| 3) | The master client sends an ss_sac_data_req APDU containing the following parameters to the slave server: - Kp_client； - CLIENT_ID； |
| 4) | The slave server checks the consistency between the received CLIENT_ID and the "authentication context". If they are consistent, calculate BCIV and BCK from the site server; If there is inconsistency, it will result in a system error. Note: The pre-stored SERVER_ID/CLIENT_ID is in the "authentication context". |
| 5) | The slave server responds with an ss_sac_data_cnf APDU containing the SERVER_ID. Failure to respond to messages will result in a system error. |
| 6) | The master client checks that the received SERVER ID matches the 'authentication context'. If the SERVER_ID returned by the main client is incorrect, a system error will be returned. |
| 7) | The master client sends an ss_sac_sync_deq APDU to indicate BCK updates. When the slave server completes the initialization of the encryption process, it sends a synchronization request to the master client, informing them that the slave server is ready to use the new CK and BCK. |
| 9) | The slave server responds with the ss_sac_sync_cnf APDU and confirms to the slave server that is also ready to use the new CK and BCK. Failure to respond to this message will result in a system error. |

The APDU format of the protocol that includes specific data types during the separate update process of BCK broadcast content keys is shown in Table 34 below:

Table 34: The process of updating BCK broadcast content keys separately includes APDUs of specific data types

| step | action | APDU | content |  |  |
| --- | --- | --- | --- | --- | --- |
| 1 | The master client sends a CLIENT_ID and a random number to the slave server | ss_sac_data_req | send _datatype_nbr=2 |  |  |
|  |  |  | i | datatype_id | datatype_len |
|  |  |  | 0 | 2(CLIENT_ID) | 64 bits |
|  |  |  | 1 | 7(Kp_client) | 256 bits |
|  |  |  | request_datatype_nbr=2 |  |  |
|  |  |  | i | datatype_id |  |
|  |  |  | 0 | 1(SERVER_ID) |  |
|  |  |  | 1 | 20(Status_field) |  |
| 2 | The slave server responds with the SERVER_ID and status information | ss_sac_data_cnf | send_datatype_nbr=2 |  |  |
|  |  |  | i | datatype_id | datatype_len |
|  |  |  | 0 | 1(SERVER_ID) | 64 bits |
|  |  |  | 1 | 20(Status_field) | 8 bits |
| 3 | The master client notifies that the calculation of the new BCK has been completed | ss_sac_sync_req |  |  |  |
| 4 | The slave server responds to its own status information | ss_sac_sync_cnf | status_field |  |  |

Note: During the content key update phase, APDU messages will encapsulate the Modbus serial link data unit field with function code 0, which is much smaller than 252 bytes (the maximum data field length of a Modbus RTU frame) of APDU. One Modbus serial link message frame can complete the interaction of APDU.

    - Content Encryption and Decryption Mechanism

After the authentication channel is established and the content keys BCK and CK are synchronized, when transmitting the encrypted Modbus serial link protocol, the encrypted content includes the original Modbus serial link protocol function code and data fields. The encryption mechanism for unicast and broadcast is the same, only the key parameters are different. The content encryption method varies depending on the synchronous encryption and decryption method. This draft recommendation supports AES GCM and SM4 GCM mode encryption algorithms, 128-EEA3/128-EIA3 encryption algorithms based on ZUC, and ChaCha20-Poly1305 encryption algorithms for reference implementation.

1) AES GCM and SM4 GCM mode encryption. Taking unicast communication as an example, the content encryption and decryption methods are shown in Figure 24 and Figure 25. The content encryption method has the following input parameters: CIV, CK, Mode, Modbus serial link protocol function code and data. Among them, CIV and CK are secret data shared by the master client and slave server after executing the content key update protocol. CIV is the CTR in the algorithm, CK is the content key, and Mode is the encryption and decryption mode agreed upon by both parties during the first binding. mode = aes_gcm or sm4_gcm, indicated by the "encryption_capability" field in the device's public key certificate. The plaintext blocks P1, P2, P3, P4, ..., Pn are 128 bit slices of data from the function and data fields in the original Modbus PDU, and those that are less than 128 bits long are filled with 0 to be 128 bit integers, with associated data taken from the first 128 bits of SM3 (“Modbus”). ENCck in Figure 24 represents the use of symmetric cryptographic algorithms such as AES/SM4 encryption algorithm to encrypt the protocol's function codes and data fields using the content key CK. Mutilh means multiplying the ciphertext block with the authentication key h. Mutilh uses polynomial multiplication, and most common microprocessors support a special polynomial multiplication instruction (CLMUL, carry free multiplication), so Mutilh has high implementation performance in both software and hardware. Mutilh is based on finite field arithmetic, specifically multiplication operations in binary Galois fields (GF (2 ^ 128)), which can provide high-level security and computational performance. The purpose of Mutilh algorithm is to generate authentication tags or message captcha (MAC) that can be used to ensure the consistency and authenticity of encrypted data. In GCM mode, Mutilh plays an important role in encrypting data authentication. The working principle of Mutilh is to process ciphertext and associated data (such as message headers or metadata) in message blocks, using binary Galois field multiplication to combine these pieces of information together. The result of the operation is XORed with the next message block, and the above steps are repeated until all message blocks are processed. As shown in Figure 25, during decryption, C1, C2, C3, C4,..., Cn represent ciphertext, and the associated data takes the first 128 bits of SM3 ("Modbus"). The receiver should first calculate and generate a MAC' using a similar encryption mechanism, and then compare it with the received MAC value to see if they are same. If they are the same, the plaintext of the content should be decrypted using the method shown in Figure 25. Otherwise, it should be considered that there is an abnormality in the current serial communication bus network, such as the possibility of a man in the middle attack.

Figure 24: Calculation steps for content encryption

Figure 25: Calculation steps for content decryption

2) 128-EEA3/128-EIA3 encryption algorithm based on ZUC. The 128-EEA3/128-EIA3 encryption algorithm based on ZUC can be found in the standards (ETSI/SAGE TS35.221, ETSI/SAGE TS35.222, GB/T 33133.1-2016, and GB/T 33133.2-2021). In this mode, the mode field corresponds to mode=zuc. The data in the original Modbus PDU is concatenated with the function and data fields and sliced into 128 bit slices. If it is not 128 bits long enough, it is filled with 0 to a 128 bit integer. Assuming that the filled data is M and the total length is MPLEN. CIV and CK are secret data shared between the master client and slave server after executing the content key update protocol. The parameter initialization corresponds to the following Table 35:

Table 35: 128-EEA3/128-EIA3 Parameter Initialization Table

| Parameter | No. of bits | Value | description |
| --- | --- | --- | --- |
| COUNT BEARER DIRECTION CK IK LENGTH IBS | 32 5 1 128 128 32 LENGTH | MSB32（CIV） 00000 0 Content key CK Content key CK MPLEN M | counter Bearing layer identification Transmission direction Confidentiality Key Integrity key The bit length of the input message stream Input message stream |

3) ChaCha20-Poly1305 encryption algorithm.

The ChaCha20-Poly1305 encryption algorithm can be found in RFC 8439. In this mode, the mode field corresponds to mode = chapoly. The data in the original Modbus PDU is concatenated with the function and data fields and sliced into 512 bit slices. If the length of slice is less than 512 bits, it is filled with 0 to a 512 bit integer. CIV and CK are secret data shared between the master client and slave server after executing the content key update protocol. The initial counter of the algorithm in this draft recommendation defaults to 0. The 256-bit encryption key, 96-bit the random number and additional data are generated as follows:

Chapoly_key = CK||CK (duplication), Nonce = MSB96(CIV), associated_data= LSB32(CIV)

ChaCha20-Poly1305 generates a 128 bit MAC while encrypting.

The encrypted Modbus security protocol data field content should be transmitted in the following APDU format, with the encapsulation method shown in Figure 26. The implementation principle of ASCII transmission mode is similar, and not repeated here:

The data content of Modbus security protocol is sent through ss_data_snd APDU, and the syntax of ss_data_snd APDU is shown in Table 36.

Table 36: ss_data_snd APDU syntax

| syntax | No. of bits | Mnemonic |
| --- | --- | --- |
| ss_data_send() { ss_data_send_tag length_field() MAC encrypted_fun_code_data } | 24 8 128 8*length | uimsbf bslbf bslbf |

ss_open_send_tag: 0x9F 90 11.

MAC: Message authentication code, if less than 128 bits, add 0, for example 128-EIA3 only outputs 32-bit MAC.

encrypted_fun_comde_data: encrypted Modbus serial link protocol function codes and data.

Figure 26: Modbus encrypted content encapsulation

Note: APDU messages will encapsulate Modbus serial link data unit fields with a success rate of 0. For APDUs larger than 252 bytes (maximum data field length of Modbus RTU frames), they can be divided into two Modbus serial link message frames for transmission. Due to the strict serial and sequential transmission operation, the upper layer application can concatenate two or more segmented APDUs based on the protocol's tag (ss_data_send_tag) and length_field () fields.

This recommendation stipulates that when the APDU responded by the slave station exceeds the single frame limit, the master station must initiate a continuous request to "read the next shard" using specific function code 0 (or its subcode/control flag) until the data length declared by length_field () is fully received. The following PDU transmission requires the same requirements.

  - Modbus Serial Link Security Protocol Based on Password or Pre-Shared Key

The key management in the Modbus serial link security protocol based on passwords or pre-shared keys relies on hardware protection. The goal of the protocol is to provide strong security on the basis of low entropy passwords, allowing master-slave devices to negotiate and obtain the same content key while resisting command or key guessing attacks. This type of protocol is suitable for use in factories and equipment with general value and security. The pre-shared key or password requires a trusted and secure execution environment protection, which shall ensure the security of user passwords or shared keys during device operation and prevent them from being stolen by malicious third parties. This model is relatively simple and can be divided into three levels: password or shared key storage level, authentication level, and content encryption level.

    - Key hierarchy structure

The password or pre-shared key storage hierarchy should include protocol public parameters, pre shared passwords or keys for the master client and slave server. The key data at this level is mainly used to execute authentication protocols. The pre-shared keys after device initialization cannot be changed or replaced, nor can they be obtained by unauthorized users. The definition of protection methods for these data is not within the scope of this draft recommendation.

At the authentication level, the master client and slave server mainly verify each other's identity through cryptographic protocols based on shared passwords or keys, and generate temporary content keys CK and an initial vector CIV for use by the content encryption layer. For the generated intermediate keys or state values, there should be a trusted execution environment to ensure security. The protection methods for these intermediate keys or state values are not within the scope of this draft recommendation. In password or pre-shared key based schemes, each party in the master-slave device combines its own password or pre shared key based value and private key with the other party's public key to obtain one or more identical keys. Other (public or private) information known to both the master and slave devices can also be used as key derivation parameters to participate in content key generation. If all parties use the same password or pre shared key and the same key derivation parameters, the protocol will ensure that all parties will receive the same content key, allowing both parties to export the shared key with only the password or pre shared key. It should be ensured that without using the password or pre shared key, neither party can successfully participate in the protocol and obtain the same content key.

The content encryption layer uses the content key CK and an initial vector CIV to encrypt and decrypt the media stream using AES/SM4. For the data encryption and decryption process of Modbus serial link communication protocol, there should be a trusted execution environment or hardware to ensure security. The definition of specific protection methods is not within the scope of this draft recommendation.

    - Authentication and Key Agreement Mechanism

This draft recommendation is based on the Modbus serial link security protocol using passwords or pre shared keys, and is based on the SM2 elliptic curve public key cryptography algorithm Part 3 Key Exchange Protocol standard developed by ISO/IEC 14888-3:2018 and China National Cryptography Administration. In this draft recommendation, the private key of the master and slave devices should be derived from a shared password or key. Assuming that the password or key shared by the master/slave devices is represented as PW, the temporary private key of the device must be generated as follows:

The master/slave devices first respectively select a random number r and calculate the 32-bit byte string PW_r=HKDF (PW, r). By converting it into an element under the field F_q of ECC, the private key d_A is obtained, and the public key P_A=[d_A] G=(x_A, y_A) is calculated. Assuming that the public and private key pairs of the master/slave devices are d_H/P_H, d_M/P_M, respectively.

The authentication and content key negotiation mechanism is shown in Figure 27 below:

s

Figure 27: Authentication and Content Key Negotiation Process Based on Password or Pre-Shared Key

The encryption algorithm mode is determined by an 8-bit mode and currently supports AES GCM mode aes_gcm, SM4 GCM mode sm4_gcm, zuc mode and chapoly mode.

The steps of Modbus serial link security protocol based on password or pre-shared key are shown in Figure 28, and the message semantics are described in Table 37:

Figure 28: Content key negotiation message exchange based on password or pre-shared key

Table 37: Content Key Agreement Message Exchange Instructions

| step | description |
| --- | --- |
| 1 | The master client sends the ss_sk_open_req APDU to the slave server. |
| 2 | The slave server responds with ss_sk_open_cnf APDU. |
| 3 | The master client sends the ss_sk_data_req APDU to the slave server. The message contains: - The master client selects the ECC point (R_H) as needed; - Encryption and decryption mode; - The ID of the requested data type. |
| 4 | The slave server responds with ss_sk_data_cnf APDU. The message contains: - The slave server selects ECC points (R_M) as needed; - Message authentication code (S_M) of the slave server; Failure to respond to this message may result in a system error, possibly due to the master client being unable to verify data from the slave server. |
| 5 | The master client then sends the ss_sk_data_req APDU. The message contains: - The message authentication code (S_H) of the master client; - Random number (r_B) used to generate broadcast content keys； Failure to respond to this message may result in a system error, possibly due to the slave server being unable to verify data from the master client. |
| 6 | The slave server responds with ss_sk_data_cnf APDU. The inability to respond to this message may result in a system error, possibly due to the inability of the master client to verify data from the slave server. |

The format definitions of ss_sk_open_req, ss_sk_open_cnf, ss_sk_data_req, and ss_sk_data_cnf APDU are the same as those of the previous ss_open_req, ss_open_cnf, ss_data_req, and ss_data_cnf APDU, respectively. But the tag types are different to distinguish different serial link content encryption mechanisms. ss_sk_open_req(tag=0x9F 90 12), ss_sk_open_cnf(tag=0x9F 90 13), ss_sk_data_req(tag=0x9F 90 14), ss_sk_data_cnf(tag=0x9F 90 15), The APDU format for protocols based on passwords or pre-shared keys that include specific data types is shown in Table 38 below:

Table 38: APDUs containing specific data type parameters in the authentication protocol

| step | action | APDU | content |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | The master client sends a ECC random point, a random number, and encryption/decryption mode to the slave server | ss_sk_data_req | send _datatype_nbr=2 |  |  |  |  |
|  |  |  | i | datatype_id |  |  | datatype_len |
|  |  |  | 0 | 22(R_H) |  |  | 512 bits |
|  |  |  | 1 | 24(mode) |  |  | 8 bits |
|  |  |  | request_datatype_nbr=2 |  |  |  |  |
|  |  |  | i | datatype_id |  |  |  |
|  |  |  | 0 | 22(R_M) |  |  |  |
|  |  |  | 1 | 25(S_M) |  |  |  |
| 2 | Send response ECC random point and authentication code from the slave server | ss_sk_data_cnf | send_datatype_nbr=2 |  |  |  |  |
|  |  |  | i | datatype_id |  |  | datatype_len |
|  |  |  | 0 | 22(R_M) |  |  | 512 bits |
|  |  |  | 1 | 25(S_M) |  |  | 256 bits |
| 3 | The master client sends a message authentication code | ss_sk_data_req | send_datatype_nbr=2 |  |  |  |  |
|  |  |  | i |  | datatype_id | datatype_len |  |
|  |  |  | 0 |  | 9(S_H) | 256 bits |  |
|  |  |  | 1 |  | 23(r_B) | 256 bits |  |

    - Content Encryption and Decryption Mechanism

The content encryption and decryption mechanism used in this section is the same as that in 9.2.5.

  - Modbus Serial Link Security Protocol Based on Post Quantum Hybrid Signature Public Key Certificate

    - Key hierarchy structure

The key management in Modbus serial link security protocol based on post quantum hybrid signature public key certificate is suitable for future high-value and high security factory equipment environments. It smoothly transitions with existing standards and is divided into three levels: certificate storage level, authentication and key negotiation level, and content encryption and decryption level. The certificate fields and contents are shown in Appendix C. Compared with the traditional ECC certificate scheme in Table 1&2, Table 39 supplements the main parameter names, storage, and usage of the post quantum algorithm involved in the post quantum hybrid signature public key certificate.

The keys stored in the certificate hierarchy mainly include ECC and post quantum algorithm public cryptographic protocol parameters, private keys of the master client and slave server, and public key certificate chains. The public key certificate chain includes root certificates, vendor certificates, and device certificates. The device certificate includes the device's ID number, ECC public key, post quantum algorithm public key, and other attribute information of the certificate. The key data at this level is mainly used to execute authentication protocols. After device initialization, the certificate must not be changed or replaced. It is recommended to use mature OTP memory based on eFuse or Anti-Use technology, OTP non-volatile memory, and data that cannot be reversibly modified after writing. The definition of protection methods for these data is not within the scope of this draft recommendation.

Table 39: Supplementary Explanation of Main System Parameters and Symbols

| Parameter | Description | Volatile or non-volatile storage | Used for exchange or local use |
| --- | --- | --- | --- |
| ML- KEM_n | ML-KEM Parameters | Non volatile storage | Local use (non replaceable) |
| ML- KEM_q | ML-KEM Parameters | Non volatile storage | Local use (non replaceable) |
| ML- KEM_k | ML-KEM Parameters | Non volatile storage | Local use (non replaceable) |
| ML-KEM_𝜂1 | ML-KEM Parameters | Non volatile storage | Local use (non replaceable) |
| ML-KEM_𝜂2 | ML-KEM Parameters | Non volatile storage | Local use (non replaceable) |
| ML-KEM_𝑑𝑢 | ML-KEM Parameters | Non volatile storage | Local use (non replaceable) |
| ML-KEM_𝑑v | ML-KEM Parameters | Non volatile storage | Local use (non replaceable) |
| MK_EK | Device’s ML-KEM public key, encapsulated key | Non volatile storage | Used for exchange (non replaceable) |
| MK_DK | Device’s ML-KEM private key, unpacking key | Non volatile storage | Local use (non replaceable) |
| ML-DSA_q | ML-DSA Parameters | Non volatile storage | Local use (non replaceable) |
| ML-DSA_𝜁 | ML-DSA Parameters | Non volatile storage | Local use (non replaceable) |
| ML-DSA_𝑑 | ML-DSA Parameters | Non volatile storage | Local use (non replaceable) |
| ML-DSA_𝜏 | ML-DSA Parameters | Non volatile storage | Local use (non replaceable) |
| ML-DSA_𝜆 | ML-DSA Parameters | Non volatile storage | Local use (non replaceable) |
| ML-DSA_𝛾1 | ML-DSA Parameters | Non volatile storage | Local use (non replaceable) |
| ML-DSA_𝛾2 | ML-DSA Parameters | Non volatile storage | Local use (non replaceable) |
| ML-DSA_(𝑘,ℓ) | ML-DSA Parameters | Non volatile storage | Local use (non replaceable) |
| ML-DSA_𝜂 | ML-DSA Parameters | Non volatile storage | Local use (non replaceable) |
| ML-DSA_𝛽 | ML-DSA Parameters | Non volatile storage | Local use (non replaceable) |
| ML-DSA_𝜔 | ML-DSA Parameters | Non volatile storage | Local use (non replaceable) |
| MK_PUBK | Device’s ML-DSA Public Key | Non volatile storage | Used for exchange (non replaceable) |
| MK_PRIK | Device’s ML-DSA Private Key | Non volatile storage | Local use (non replaceable) |

For master-slave devices, MK-DK and MK-PRIK are the main private keys for both parties. MK-DK is mainly used for key encapsulation, while MK-PRIK is the main private key for the signing process, which must be ensured not to be modified or intercepted by third parties. It is recommended to use a mature TEE environment. The definition of protection methods for MK-DK and MK-PRIK keys is not within the scope of this draft recommendation.

    - Authentication and Key Agreement Mechanism

The key management in Modbus serial link security protocol based on post quantum hybrid signature

The authentication and key negotiation based on post quantum hybrid signature PKI public key certificate includes certificate verification, generation of CK and BCK keys. Firstly, in the first stage, the authentication process between the master client and the slave server begins by exchanging their respective certificate chains rooted in ROT. The master client must verify the certificate chain of the slave server, and similarly, the slave server must also verify the certificate chain of the master client. During the verification process, the slave server will extract the CLIENT ID (64 bit data) from the master client's public key certificate, and similarly, the master client will also extract the SERVER ID (64 bit data) from the slave server's public key certificate. The verification priority for certificate chains with mixed signatures is as follows. If all certificates on the certificate chain have ECC parameters that meet legal requirements, they should be processed directly according to the protocol in Section 9.2. If ECC parameters are non-compliant, such as key length non-compliance or mandatory use of post quantum algorithms, they should be handled according to post quantum mechanisms. If post quantum related parameters are also non-compliant, the current certificate chain of the device should be considered non-compliant.

In the first stage, the slave server and master client based on post quantum algorithms must perform a key encapsulation exchange mechanism using device certificates to obtain a long-term secret master key KEMSK and a long-term authentication key (which can reduce the first stage of public key authentication operations when the master and slave devices are powered on for re authentication, as described in the "Authentication Context and Re authentication" section below). Subsequently, both parties should generate temporary content encryption keys based on KEMSK. Both parties should store the master key KEMSK and authentication key in NVM and keep them confidential for subsequent Modbus protocol data communication encryption key derivation.

In the second stage, the slave server will send a request to the master client to confirm the authentication key. After receiving the confirmation message, the slave server must compare the received key value with the stored key value. If the two are consistent, it can prove that the master client is legitimate.

The authentication and key negotiation steps based on post quantum hybrid signature PKI public key certificate are shown in Figure 29, and the message semantics are explained in Table 40:

Figure 29: Authentication and Key Agreement Message Exchange Based on Post Quantum Certificate

Table 40: Explanation of Authentication and Key Agreement Message Exchange Based on Post Quantum Certificate

| No. | Description |
| --- | --- |
| 1 | The slave server sends ss_open_req APDU to the master client. |
| 2 | The master client responds with ss_open_cnf APDU. |
| 3 | Send ss_data_req APDU from the slave server to the master client. The message contains: - Device certificate (Server_dev_Cert) of slave server; - Brand certificate (Server_Brand_Cert) of slave server; - ID of the requested data type. |
| 4 | The master client responds with ss_data_cnf APDU. The message contains: - Key encapsulated message ciphertext (KEM-C) of the master client; - Key encapsulated message ciphertext (KEM-BC) of the master client; - Encryption and decryption mode - Device certificate (Client_Dev_Cert) of master client; - Brand certificate (Client_Brand_Cert) of master client. Failure to respond to this message may result in a system error, possibly due to the master client being unable to verify data from the slave server. |
| 5 | The slave server sens ss_data_req APDU to the master client. The message contains: - The data type ID of the requested nonce as Challenge. |
| 6 | The master client responds with ss_data_cnf APDU. The message contains an SM3(AKH, Challenge), Either it is valid or it is all zero, indicating illegality. Failure to respond to this message will result in a system error. |
| 7 | The slave server calculates and compares the response values received locally from the station. If there is inconsistency, it will result in a system error. |

The APDU format of the protocol based on post quantum certificate authentication and key agreement protocol, which includes specific data types, is shown in Table 41 below:

Table 41: APDUs with Specific Data Type Parameters in the Authentication and Key Agreement Protocol Based on Post Quantum Certificates

| Step | action | APDU | content |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | Send public key certificate chain from the slave server to the master client | ss_data_req | send _datatype_nbr=2 |  |  |  |  |  |
|  |  |  | i |  | datatype_id |  | datatype_len |  |
|  |  |  | 0 |  | 11(Server_Dev_Cert) |  | 1280bits |  |
|  |  |  | 1 |  | 4(Server_Brand_Cert) |  | 1280 bits |  |
|  |  |  | request_datatype_nbr=5 |  |  |  |  |  |
|  |  |  | i |  | datatype_id |  |  |  |
|  |  |  | 0 |  | 26 (KEM_C) |  |  |  |
|  |  |  | 1 |  | 27 (KEM_BC) |  |  |  |
|  |  |  | 2 |  | 24 (mode) |  |  |  |
|  |  |  | 3 |  | 10 (Client_Dev_Cert) |  |  |  |
|  |  |  | 4 |  | 3 (Client_Brand_Cert) |  |  |  |
| 2 | The master client sends KEM-C, KEM-BC,encryption mode and two public key certificates | ss_data_cnf | send_datatype_nbr=5 |  |  |  |  |  |
|  |  |  | i |  | datatype_id |  | datatype_len |  |
|  |  |  | 0 |  | 26 (KEM_C) |  | 8704 bits |  |
|  |  |  | 1 |  | 27 (KEM_BC) |  | 8704 bits |  |
|  |  |  | 2 |  | 24 (mode) |  | 8 bits |  |
|  |  |  | 3 |  | 10(Client_Dev_Cert) |  | 1280bits |  |
|  |  |  | 4 |  | 3(Client_Brand_Cert) |  | 1280 bits |  |
| 3 | Request AKH from the slave server to the master client | ss_data_req | request_datatype_nbr=1 |  |  |  |  |
|  |  |  | i |  | datatype_id |  |  |
|  |  |  | 0 |  | 17(AKH) |  |  |
| 4 | The master client sends AKH | ss_data_cnf | send_datatype_nbr=1 |  |  |  |  |
|  |  |  | i | datatype_id |  | datatype_len |  |
|  |  |  | 0 | 17(AKH) |  | 256 bits |  |

The ss_open_req and ss_open_cnf APDU are the same as the ones in Section 9.2.2.

Note: During the certificate authentication phase, APDU messages will encapsulate the Modbus serial link data unit field with a success number of 0. For APDUs larger than 252 bytes (the maximum data field length of Modbus RTU frames), such as ss_data-req APDUs, which exchange certificates with approximately 422 bytes, they can be divided into two Modbus serial link message frames for transmission. Since the transmission operation is strictly serial and has a sequence, the upper layer application can concatenate two or more segmented APDUs based on the tag (ss_data_cnf_tag) and length_field () fields of the protocol.

The calculations involved in the authentication and key negotiation process based on post quantum certificates are shown in Figure 30, with specific explanations provided in Table 42.

Figure 30: The computational steps involved in the authentication process based on post quantum certificate

Table 42: Explanation of Calculations Involved in the Authentication Process Based on Post Quantum Certificate

| No. | Description |
| --- | --- |
| 1) | The slave server sends its public key certificate chain to the master client |
| 2) | The master client verifies the validity of the two received certificates from the slave servers. |
| 3) | The master client uses appropriate PRNG to generate two 32 byte random numbers, and calculates and obtains a master key KEMSK and a broadcast master key KEMSK_B according to the ML-KEM.Encaps algorithm, as well as their corresponding encapsulated key ciphertexts KEM-C and KEM-BC. |
| 4) | The master client sends the encapsulated key ciphertexts KEM-C, KEM-BC, as well as the master client device certificate and the brand certificate of the master client device manufacturer to the slave server. |
| 5) | The slave server verifies the validity of the received two certificates. |
| 6) | The slave server verifies the parameters, use KEM-C and ML-KEM.Decaps algorithms to calculate the 32 byte master key KEMSK and broadcast master key KEMSK_B, respectively, and calculate and store the authentication key (AKM). |
| 7) | The master client calculates and stores the authentication key (AKH). |
| 8) | The slave server start to enter the second stage of the authentication process: request the current authentication key AKH from the master client through the response APDU. |
| 9) | The master client uses the corresponding APDU to send AKH to the slave server. |
| 10) | The slave server compares the received AKH with its own calculated AKM. If they are different, it will result in a system error. |

Step 1: Calculation of authentication keys (AKH and AKM), which can be used in subsequent authentication processes. The authentication key is generated when the master client and slave server are first bound and stored in NVM, which can be used for repeated authentication in the future. The calculation process is as follows:

AKM = AKH = HKDF(KEMSK, SERVER_ID, CLIENT_ID, ss_system_id_bitmask, mode)

Step 2: Export CK and CIV.

CK = MSB128(HKDF(KEMSK, SERVER_ID)), CIV = LSB128(HKDF(KEMSK, SERVER_ID))

Step 3: Export BCK and CIV, and use the broadcast master key KEMSK_B for broadcast communication between the master client and all sdlave servers.

BCK = MSB128(HKDF(KEMSK_B, CLIENT_ID)),

BCIV = LSB128(HKDF(KEMSK_B, CLIENT_ID))

Authentication context and re-authentication: Authentication context is a secure data object stored in NVM with a validity period, and its storage format is not strictly defined in this document. After the expiration date, it is generally considered that the context is not trustworthy and needs to be deleted and a new binding relationship needs to be established in the future. However, in the implementation of actual products, it should be given sufficient security to ensure that it is not changed by attackers. The authentication context is used to store the results of public key authentication between both parties of the device, and is used to perform the re authentication process. The information that should be included in the authentication context of the slave server and master client is shown in Table 43.

Table 43: Information in Authentication Context

| The information that should be included in the authentication context |  |
| --- | --- |
| Slave server | Master client |
| CLIENT_ID KEMSK AKM Encryption mode | SERVER_ID KEMSK AKH Encryption mode |

After the master client and slave server are first bound, the device should associate the DHSK and authentication key negotiated during the public key authentication process with an authentication context. The master client should support at least 247 authentication contexts, and the slave server should support at least 1 authentication context.

After establishing a session, the master client and slave server can first perform an authentication key verification step to check if there is an existing binding relationship between the two devices. If there is, there is no need to perform a public key authentication protocol. If the slave server has a valid authentication context, it should directly request the master client to send an AKH and then check if the received AKH matches the AKM in the local authentication context. If there is inconsistency, the slave server shall attempt up to 246 more times (as the master client may have 247 authentication contexts). When there are no other available authentication contexts for the master client, the master client should send a zero value as a response to the AKH request from the slave server, and the slave server should start executing the public key authentication protocol. If the master client has the authentication context of the slave, it should first send the AKH in the context of the slave. If the AKH is inconsistent with the AKM of the slave server, the slave server should attempt to request AKH again, and the master client should send other AKHs in other authentication contexts. So, re-powering on the slave server may not necessarily execute the public key authentication protocol to reduce authentication time. But when the slave server accesses an unbound master client, or when the master client accesses an unbound slave server, the identity authentication protocol related to the public key certificate is required to be executed.

    - Content Encryption and Decryption Mechanism

The content encryption and decryption mechanism used in this section is the same as that in 9.2.5.

- Security requirements

- The security extension protocol specification should comply with " IEC 62443-3-3:2013: System Security Requirements and Security Levels";

- The security extension protocol specification should comply with " ANS1/ TIA/ EIA-232-E-1997 Interface Between Data Terminal Equipment and Data Circuit-Terminating Equipment Employing Serial Binary Data Interchange";

- The security extension protocol specification should comply with " Modbus Security Specification: Modbus Application Protocol Specification ";

- The security extension protocol module, including integrated embedded security management module, external module, or security proxy module, should have tamper proof and physical protection design, including voltage/temperature sensors, fuse technology, side channel attack protection, etc;

- The security extension protocol module should support hardware root of trust;

- The security extension protocol module should support secure storage and key management, where keys are generated, used, and destroyed within the HSM and never exposed in plaintext. It should also support key rotation, revocation, and hierarchical management;

- The security extension protocol module should support binding HSM hardware features with unique device identifiers; ‌

- The security extension protocol module should support hardware encryption based runtime integrity monitoring (RIM) technology to detect program execution status, and be able to combine abnormal log analysis to achieve rapid response and ensure the trusted execution of critical functions; ‌

- The security extension protocol module should support dedicated security chips (supporting TPM or SE), encryption modules, secure boot mechanisms, trusted execution environments (TEE), secure firmware update mechanisms, and hardware isolation technologies;

- The security extension protocol module should support audit functionality to trace the software's operating status. For example, in industrial control systems, timestamp based audit logs are used, combined with blockchain technology to achieve tamper proof logs. By mining audit data, potential security threat patterns can be identified.

- The secure boot mechanism of the security extension protocol module should comply with the IEEE1801-2018 "Scalable Firmware Interface Standard";

- The TEE of the security extension protocol module should comply with ISO/IEC 21827-1:2017 "Information Security Technology - Information Security Management System";

- The security extension protocol module should support identity authentication and data protection through hardware level encryption algorithms and non-volatile storage;

- The use of the security extension protocol module should meet the security level classification and technical requirements of the specified password module (including HSM) according to different industrial application scenarios, and match the information system security level protection (equivalent protection);

- The security extension protocol module should use a hardware encryption module to verify firmware signatures, support multi-level access control, firmware version verification and rollback functions, and comply with the "SJ/T 11368-2018 Embedded System Security Technical Requirements";

- The security extension protocol module should use a random number generator that complies with FIPS-140-2 to ensure that the internal true random number generator (TRNG) meets security requirements.

- Security considerations

Contributor’s Note: The security considerations of  this draft recommendation except for the general security issues of trustworthy networking will be addressed in this clause, contributions are welcomed.

This draft recommendation defines the reference native security protocol specification for the case with pure serial link communication, which (e.g., RS485-based Modbus) remains an unresolved challenge. Serial Modbus transmits data in plaintext, leaving it susceptible to interception, modification, and hardware-based side-channel attacks. There are currently no formal standards addressing security for Modbus over RS485, which is still widely used due to its simplicity and long-distance capabilities. Currently, for any EIA/TIA-485 multi-point system, whether it is a 2-wire or 4-wire configuration, the maximum length for cables with a maximum baud rate of 9600bit/s and AWG26 (or thicker) specifications can reach over 1000m. For RS485 Modbus, a sufficiently wide cable diameter should allow for a maximum length of over 1000m, and for RS485 Modbus using Category 5 cables, the maximum length can reach 600m. This lack of encryption and authentication mechanisms poses risks, particularly in scenarios involving relays or bridged networks. While Modbus TCP has seen notable advancements in security through TLS-based solutions, serial Modbus communication requires more attention. Introducing lightweight encryption and authentication mechanisms for serial links could provide a practical way to enhance security and protect legacy systems without significant infrastructure changes. Therefore, it is valuable and necessary to improve and enhance the communication security of Modbus protocol under serial link mode, for reference by relevant institutions and organizations in various industries, in order to ensure the practical application security of various industrial control systems. With the introduction of the Modbus serial link security standard, the introduction of encryption and authentication mechanisms in the serial link communication channel of the Modbus protocol significantly improves its security, providing a relatively simple and direct upgrade path for existing devices that use Modbus extensively.

_______________________

- Bibliography

[b-Langley] 	Langley, A. (2015). "Maintaining Digital Certificate Security." Google Security Blog.

[b-ITU-T X.1145] 	Recommendation ITU-T X.1145 Security framework and requirements for open capabilities of telecommunication services.

[b-ITU-T X.1219] 	Recommendation ITU-T X.1219 Functional requirements for a secured process to evaluate technical vulnerabilities.

[b-ITU-T X.500] 	Recommendation ITU-T X.500|ISO/IEC 9594-1, Information Technology-Open System Interconnection-The Directory: Overview of concepts, models and services.

[b-ITU-T X.501] 	Recommendation ITU-T X.501|ISO/IEC 9594-2, Information Technology-Open System Interconnection-The Directory: Models.

[b-ITU-T X.509] 	Recommendation ITU-TX.509|ISO/IEC 9594-8, Information Technology-Open System Interconnection-The Directory: Public-key and attribute certificate framework.

[b-ITU-T X.510] 	Recommendation ITU-TX.509|ISO/IEC 9594-11, Information Technology - Open systems Interconnection - The Directory - Protocol specifications for secure operations.

[b-ISO/IEC 11770-4:2006] 	ISO/IEC 11770-4:2006 Information technology — Security techniques — Key management — Part 4: Mechanisms based on weak secrets.

[b-ISO/IEC 14888-3:2018] 	ISO/IEC 14888-3:2018 IT Security techniques — Digital signatures with appendix — Part 3: Discrete logarithm based mechanisms.

[b-IEEE Std 1363.2-2008] 	IEEE Std 1363.2-2008 IEEE Standard Specifications for Password-Based Public-Key Cryptographic Techniques.

[b-IETF RFC8446] 	RFC8446 The Transport Layer Security （TLS） Protocol Version 1.3.

[b-IETF RFC5246] 	RFC5246 The Transport Layer Security （TLS） Protocol Version 1.2.

[b-IETF RFC6066] 	RFC6066 Transport Layer Security （TLS） Extensions: Extension Definitions.

[b-IETF RFC6961] 	RFC6066 The Transport Layer Security （TLS） Multiple Certificate Status Request Extension.

[b-ETSI/SAGE TS35.221]	ETSI/SAGE TS35.221 Specification of the 3GPP Confidentiality and Integrity Algorithms 128-EEA3 &.128-EIA3.Document 1:128-EEA3 and 128-EIA3 Specification.

[b-ETSI/SAGE TS35.222]	ETSI/SAGE TS35.222 Specification of the 3GPP Confidentiality and Integrity Algorithms 128-FEA3 & 128-EIA3.Document 2: ZUC Specification.

[b-GB/T 33133.1-2016]	GB/T 33133.1-2016 Information security technology ZUC stream cipher algorithm Part 1: Algorithm description.

[b-GB/T 33133.2-2021]	GB/T 33133.2-2021 Information security technology ZUC stream cipher algorithm Part 2: Confidentiality algorithm.

[b-GB/T 33133.2-2021]	GB/T 33133.2-2021 Information security technology ZUC stream cipher algorithm Part 3: Integrity algorithm.

- Annex III

Parameter datatype id table exchanged in APDU.

Table A.1 summarizes the parameters and data type ID number used in this draft recommendation.

Table A.1: System parameter numbers in secure communication protocol calculation

| Keywords or variables | No, of bits | description | datatype id |
| --- | --- | --- | --- |
| CLIENT_ID | 64 | Generated by ROT and included in X.509 certificate. | 1 |
| SERVER_ID | 64 | Generated by ROT and included in X.509 certificate. | 2 |
| Client_Brand_Cert | variable size | Master server brand certificate | 3 |
| Server_Brand_Cert | variable size | Slave client brand certificate. | 4 |
| Reserved | - | Reserved. | 5 |
| Reserved | - | Reserved. | 6 |
| Kp | 256 | Key parameters for updating content keys of master and slave device. | 7 |
| DHPH | 512 | Master device’s DH public key. | 8 |
| DHPM | 512 | Slave device’s DH public key. | 9 |
| Client_dev_Cert | variable size | Master device’s device certificate. | 10 |
| Server_dev_Cert | variable size | Slave device’s device certificate. | 11 |
| RM(Signature_A) | 256 | Signature of DH public key on the master client. | 12 |
| RH(Signature_B) | 256 | Signature of DH public key on the slave server. | 13 |
| auth_nonce | 256 | The 256 bit random number generated by the slave server is transmitted from the slave server to the master client for authentication protocol. | 14 |
| Ns_H | 64 | Challenge parameters of SAC from master client to slave server. | 15 |
| Ns_M | 64 | Challenge parameters of SAC from slave server to master client. | 16 |
| AKH | 256 | Master client’s authentication key | 17 |
| AKM | 256 | Slave server’s authentication key | 18 |
| Reserved | - | Reserved. | 19 |
| status_field | 8 | Confirmation message for status field in APDU. | 20 |
| Reserved | - | Reserved. | 21 |
| R_H/R_M | 512 | Randomly selected point on ECC. | 22 |
| r_B | 256 | A random number used to generate broadcast content keys. | 23 |
| mode | 8 | Encryption and decryption algorithm mode | 24 |
| S_M/S_H | 256 | Message authentication code | 25 |
| KEM_C | 8704 | Content key encapsulation value | 26 |
| KEM_B | 8704 | Broadcast content key encapsulation value | 27 |
| Challenge_response | 256 | 256 bits challenge | 28 |

- Annex IV

Pseudo-random number generator

The random numbers generated by the pseudo-random number generator PRNG in this draft recommendation are shown in Table B.1.

Table B.1: Parameters for Using Pseudo Random Number Generator

| random number | No, of bits | description |
| --- | --- | --- |
| DHX | 256 | DH exponent "x" |
| DHY | 256 | DH exponent "y" |
| Kp | 256 | Used for calculating CK |
| Ns_H | 64 | Used to establish SAC |
| Ns_M | 64 | Used to establish SAC |
| r_B | 256 | Used to generate broadcast content keys. |

This draft recommendation requires the use of FIPS-140-2 and a pseudo-random number generator defined by [SCTE 41:2004], section 4.6, RGB required by NIST.FIPS.203, NIST.FIPS.204 and An AES based pseudo-random number generator by ANSI X 9.31 1316H [ANSI X 9.31:1998].

- Annex V

Trust Model and Certificate Format

1) Trust Model

The Modbus serial link communication security protocol based on traditional ECC PKI public key certificate adopts a tree shaped PKI structure, as shown in Figure V.1. The system has only one trust root (ROT), which issues public key certificates to industrial equipment manufacturers of different brands, and can also issue public key certificates to different business operators (factories). In addition, ROT also issues equipment certificates for different brands of industrial equipment manufacturers for their respective equipment brands. In actual operation, these brand certificates and equipment certificates can be issued by the original factory in the product at the time of leaving the factory, or they can be deployed and operated in the product by third-party agents or the most network user factory after leaving the factory.

The trust model structure based on post quantum hybrid signature PKI public key certificate is the same as the tree structure of traditional ECC PKI method, but the difference is that ROT will have two independent certificates generated by ECC algorithm and post quantum algorithm respectively.

The brand certificate contains the signature public keys required for ECC algorithms such as SM2 and post quantum signature algorithms such as ML-DSA, which consist of two public keys that can be concatenated and stored in the subjectPublicKeyInfo field, AlgorithmIdentifier is defined as SM2withSM3+ML-DSA, or they can be stored separately in the subjectPublicKeyInfo field and the extension field of the main public key information. The extension field includes Key Usage: Digital Signature, id-oqs-public-key: ML-DSA Public Key，id-oqs-algorithm: ML-DSA. In addition, the brand certificate also includes corresponding signatures generated from two ROT certificates and combined in a single signature format in the Signature field of the certificate (i.e. containing two signatures). The specific storage form is not mandatory in this draft recommendation.

The device certificate contains the public key required for ECC algorithms such as SM2 and the public key required for post quantum key encapsulation algorithms such as ML-KEM, as well as signatures generated by the corresponding brand certificate through ECC algorithm and ML-DSA algorithm private key respectively, and combined in a single combination signature in the Signature field of the certificate (i.e. containing two signatures). The specific storage form is not mandatory in this draft recommendation.

Based on post quantum hybrid signature PKI public key certificate, referring to NIST recommendations and considering protocol application scenarios, this draft recommendation recommends KEM algorithm to support ML-KEM-768, while DSA signature algorithm is recommended to support ML-DSA-44.

Figure V. 1: Trust Model

2) Certificate format and content

The Modbus serial link communication security protocol based on ECC public key certificates has three types of public key certificates: ROT root certificate, manufacturer brand certificate, and device certificate. Each device stores the root certificate of ROT, device certificate, and the corresponding manufacturer's brand certificate in its NVM. The root certificate is a self-signed certificate used to verify the manufacturer's brand and device certificate. The manufacturer's brand certificate is issued by ROT and is used to verify the equipment certificate. These three types of certificates have a unified field format, following the X.509 format. The basic and extended fields they contain are shown in Table C.1. They are distinguished by the "Certificate Type" field in the certificate.

Table C.1: ECC Public Key Certificate Fields and Structure

| syntax | No. of bits | Mnemonic |
| --- | --- | --- |
| Modbus_certificate ( ) { modbussec_ID certificate_type certificate_version encryption_capability holder_ID holder_public_key_x holder_public_key_y issuer_ID signed_date reserved signature_c signature_d } | 8 8 8 8 64 256 256 64 32 64 256 256 | bslbf bslbf bslbf bslbf bslbf uimsbf uimsbf bslbf bslbf bslbf uimsbf uimsbf |

Modbussec-ID: This field represents the security system to which the certificate belongs, and this section specifies its value as 0xC2.

Certificate type: This field represents the type of certificate. See Table C.2.

Table C.2: Certificate Types

| certificate_type | the type of certificate represented |
| --- | --- |
| 0x00 0x01 0x02 0x03 Others | Root certificate Operator Certificate Brand Certificate Device certificate Reserved |

Certificate_version: This field represents the version of the certificate, and this section specifies that the version number for all types of certificates is 0x01.

Encryption_capability: This field represents the encryption modes supported by the device, and AES algorithm is used by default, as shown in Table C.3.

Table C.3: Encryption and Decryption Capability

| encryption_capability | description |
| --- | --- |
| 0x00 0x01 0x02 0x04 0x08 Others | Reserved Used for device certificate, indicating that the device supports aes_gcm mode Used for device certificates, indicating that the device supports sm4_gcm mode Used for device certificates, indicating that the device supports zuc mode Used for device certificates, indicating that the device supports chapoly mode Reserved |

Holder_ID: This field represents the ID of the certificate holder, which is SERVHER-ID or CLIENT-ID.

Holder_public_key_x: This field represents the horizontal axis value of the certificate holder's public key.

Holder_public_key_y: This field represents the y-axis value of the certificate holder's public key.

Issue_ID: This field represents the ID of the certificate issuer, which is the holder ID in the certificate issued by the CA that issued the certificate.

Signed_time: This field represents the date of issuance of the certificate. This time is composed of the lower 16 bits of MJD time and four 4-bit BCD digits. For example, on October 13, 1993, 12:45 was encoded as "0xC0791245".

Reserved: Set all fields to zero for future expansion.

Signature_c: This field represents the c value of the certificate issuer's digital signature on the certificate data.

Signature-d: This field represents the d value of the digital signature of the certificate issuer on the certificate data.

Note: The certificate signature scheme adopts ECSSA scheme in 10.2 of IEEE 1363-2000 standard, the signature algorithm adopts ECSP-DSA algorithm in 7.2.7 of IEEE 1363-2000 standard, the message encoding method adopts EMSA1 method in 12.1.1 of IEEE 1363-2000 standard, and the hash function in the encoding method adopts SHA-256. Alternatively, the SM2 and SM3 algorithms in the ISO/IEC 14888-3:2018 standard can be used.

Note: The verification algorithm corresponding to the above signature algorithm is the ECVP-DSA algorithm or SM2 and SM3 algorithms in 7.2.8 of the IEEE 1363-2000 standard.

The PKI public key certificate based on post quantum hybrid signature follows the X.509 format, which includes basic and extended fields as shown in Table C.4.

Table C.4: PKI Public Key Certificate Fields and Structure Based on Post Quantum Hybrid Signature

| syntax | No. of bits | Mnemonic |
| --- | --- | --- |
| Modbus_certificate ( ) { modbussec_ID certificate_type certificate_version encryption_capability holder_ID holder_public_key_x holder_public_key_y ml_dsa_public_key(For brand certificate) ml_kem_public_key(For device certificate) issuer_ID signed_date reserved signature_c signature_d signature_𝜎 } | 8 8 8 8 64 256 256 1312*8 1184*8 64 32 64 256 256 2420*8 | bslbf bslbf bslbf bslbf bslbf uimsbf uimsbf uimsbf uimsbf bslbf bslbf bslbf uimsbf uimsbf uimsbf |

ml_desa_public_key: This field represents the ML-DSA algorithm public key of the certificate holder, which is only applicable to brand certificates.

ml_kem_public_key: This field represents the ML-KEM algorithm public key of the certificate holder, which is only applicable to device certificates.

signature_𝜎: This field represents the ML-DSA algorithm signature in the mixed signature of the certificate.

The meaning of other fields is the same as mentioned above.

3) Certificate holder ID

In order to support different revocation granularity requirements in the future, when the certificate type is "device certificate", further definition of the holder_ID domain is required, as shown in Figure C.2.

  - Define the top 16 bits in the "holder-ID" field (64 bits) as the " device brand identifier";

  - Define the lower 32 bits of the "holder-ID" field (64 bits) as the "device serial number";

  - Define the middle 16 bits of the "holder-ID" field (64 bits) as the "device model identifier".

Figure C.2: Structure of Certificate Holder ID

In addition, this section requires that when the certificate type is "device certificate", the values of “device brand identifier" and "device model identifier" cannot be zero; The " device brand identifier" and "device model identifier" in the IDs of the other types of certificates are both set to zero values.

4) Elliptic curve parameters

The Modbus serial link communication security protocol based on public key certificates in this draft recommendation uses the elliptic curve defined in the prime field according to the SM2 standard, where:

EC_p=0x8542D69E 4C044F18 E8B92435 BF6FF7DE 45728391 5C45517D 722EDB8B 08F1DFC3

EC_a=0x787968B4 FA32C3FD 2417842E 73BBFEFF 2F3C848B 6831D7E0 EC65228B 3937E498

EC_b=0x63E4C6D3 B23B0C84 9CF84241 484BFE48 F61D59A5 B16BA06E 6E12D1DA 27C5249A

The order of the above curve is:

EC_N=0x8542D69E 4C044F18 E8B92435 BF6FF7DD 29772063 0485628D 5AE74EE7 C32E79B7

The coordinates of the base point EC_G=(Gx, Gy) on the curve:

Gx=0x421DEBD6 1B62EAB6 746434EB C3CC315E 32220B3B ADD50BDC 4C4E6C14 7FEDD43D

Gy=0x0680512B CBB42C07 D47349D2 153B70C4 E5D7FDFC BFA36EA1 A85841B9 E46E09A2